FluidNinja LIVE	Manual

Document updated:	5 Oct 2021

Latest NinjaLive version:	1.5.23.3	(for UE 4.23 - 4.25)

1.5.26.3	(for UE 4.26 - 4.27)

NinjaLive is providing real   me, responsive fluid simula  on inside Unreal. For baking fluid simula  on, see: NinjaTools

Support:	andras.ketzer@gmail.com

Discord:	h  ps://discord.gg/VpcyBQa77w

Updates, news:	h  ps://twi  er.com/FluidNinjaLIVE

Project home page:	unrealengine.com/marketplace/fluidninja-live

Changelog, Issues, FAQ

h  ps://drive.google.com/file/d/17oVPVEoaW6Y6YKNISr4S0uUJY4_Yx_FM

(1) Showcase videos, (2) Tutorials, (3) Use cases:

h  ps://www.youtube.com/playlist?list=PLVCUepYV6TvOrOfQVLMCxl_JoU_cIkK8P h  ps://www.youtube.com/playlist?list=PLVCUepYV6TvPYbofpEf_ghznfihM-yt-B

h  ps://www.youtube.com/playlist?list=PLVCUepYV6TvO6QhpqaT1GRVGC96QMcUJs

Document share URL:

h  ps://drive.google.com/file/d/1I4dglPjeXLcNkSGxGok8sQCy59qgYcF9

Contents

0. Step-by-step Guides / Tutorial Videos

1. Introduc  on

1.1 Inputs

1.2 Level & Game Design

1.3 Rendering

2. Implementa  on

11. U  li  es (PossessPawn, FPS limiter)

12. Performance Tests

13. Turning Ninjalive To A Plugin

14. Packaging / Compiling In General

15. Packaging To Android, Briefly

16. Planned Features

17. Op  miza  on

3. Loca  on & Size

4. Versions - UE4, UE5 compa  bility

5. Usage

6. Core Func  ons & Terminology

18. Changelog & Updates

19. Known Issues & FAQ

20. Niagara

outsourced, see LINK outsourced, see LINK

6.1 NinjaLiveComponent

6.2 NinjaLive

6.3 Actor Details vs Actor Component Details

6.4 Preset Manager and Presets

6.5 Simple Painter Mode

7. Memory Demand

8. Performance (Excerp  on)

9. High Level Se   ngs: Merging NinjaLive to your Project

10. Low Level Se   ngs

10.1 NinjaLive Actor Params (briefly)

10.2 NinjaLive Actor Component Params (briefly)

10.3 Adding NinjaLiveComponent to your own Actor classes

10.4 Other Ways To Include/embed/add Ninjalive To Actors

10.5 Interac  on: se   ng up Objects to trigger NinjaLive response

21. Complete List of NinjaLive parameters

21.1 GUI exposed params

21.2 NinjaLive Actor params

21.3 NinjaLive Actor Component params

22. Fluidsim pipeline: a technical descrip  on

23. Sequencer & Movie Render Queue

24. Volumetrics

25. Detail Maps

26. Controlling Ninja In Real Time

27. Mapping sim on 3D mesh surfaces

28. Moving in WorldSpace

29. Addi  onal Content

Step by Step guides in this Manual

Topics

Merging NinjaLive to your project

Adding NinjaLive Component To Your Own Actor Classes Linking Niagara and Ninja

Cinema  cs Rendering via Movie Render Queue Linking Volumetrics And Ninja

(Re)configuring a sim container to use 3D sim mesh

Chapter

9 10.3 20.3 23.3 24.4 27.1

Tutorials on YouTube

Topics

Tutorial videos playlist Showcase videos playlist

Introduc  on

Merging NinjaLive to your project

Changing default (planar) TraceMesh to Custom Mesh

Use TraceMesh to detect overlap (instead of Interac  onVol.) User defined line-tracing source   (instead of Camera)

URL

LINK

LINK

LINK LINK LINK LINK LINK

Render Buffers & Output Materials	LINK

Disable Dynamic Asset Lookup (DAL), in the FAQ doc

i005

Sim   ling / wrapping

Pain  ng persistent Flow pa  erns (2D, Fog, Clouds...etc)

LINK1, LINK2 LINK

Using Sequencer to animate Objects and Sim Params	LINK

Character FX related videos:

Adding NinjaLiveComponent to Your Character

LINK

SimDetails, CameraFacing, SingleTargetMode	LINK

Reposi  oning TraceMesh, adding Custom Sockets	LINK

Mul  ple fluidsim components in a single character	LINK

Hands: two dedicated sim components tracking all fingers	LINK

Introduc  on

FluidNinja LIVE is providing real   me, responsive fluid simula  on for PC and Mobile app developers, inside Unreal. Live is focusing on performance, backward compa  bility and simple use. It could be u  lized in any Unreal project: following a 5 minute setup, responsive fluid simula  on Actors could be placed on level. Live provides an ActorComponent too, that could be added to user defined classes, pawns, vehicles. Live is easy to customize, features preset-based fluid simula  on management, exposed variables and annotated blueprints, user editable output materials and 34 demo levels providing usage examples. Live is scalable: could be op  mized for average mobile devices - or run a 4k simula  on container. Lowest supported UE version is 4.20. Live does not rely on Niagara - but could be used to drive Niagara.

1.1 Inputs

Live simula  on could be driven by (a) Textures [sta  c], (b) Materials [animated], (c) SceneCapture camera and Streaming video, (d) Scene geometry - including Sta  c and Dynamic meshes, Skeletal Meshes (Bones, Sockets), PhysicsBodies. Plus user gestures via mouse, touchscreen or other device.

1.2 Level & game design

Live offers LOD, Proximity based Sleep/Wake and Memory Pooling - suppor  ng a design approach with mul  ple, local fluidsim containers for character and area effects. Live is not a robust "whole world system", but a system suppor  ng a mul  tude of dispersed, local interac  ons.

1.3 Rendering

Live uses 2D fluid simula  on, combined with technology that makes it behave like a volume.

A. Overlap Projec  on is using a volume to track objects in the sim area, mapping 3D

posi  onal and velocity data to the 2D sim using Line Trace from the camera (or from a user defined point). As a result, moving objects behind and in front of the simula  on plane could also affect the sim - as if it would be spa  ally extended.

B.  Op  onally, overlap projec  on could be further improved by running the sim on a camera facing plane: viewers could walk around the interac  on volume, while the pa  erns generated by overlapping objects appear in the correct spa  al posi  on.

C.  The moving direc  on of overlapping objects is handled in 3D by the velocity solver: the perpendicular component of mo  on (Z) causes expansion / shockwaves, while components aligning with the 2D simula  on plane (X,Y) are dragging the fluid.

D. 3D iner  al frame of reference is supported: in case the whole sim area is moving, the

fluid "lags behind" as Live channels world-space accelera  on to the simula  on.

E.  Raymarching: light sources in 3D space (both sta  c and moving) could generate self

shadows on the fluid with a proper direc  on.

F.  Parallax Mapping is further enhancing this effect by using fluid density as a height map

to offset sim data, making it look spa  ally extended.

G. A well-balanced configura  on of noises and a custom pressure solver helps to keep the

number of fluidsim pressure itera  ons low, s  ll producing detailed flow with vor  ces.

In the end, we have dynamic fluid in 3D - with the GPU demand of a 2D sim.

2. Implementa  on

NinjaLive is a compact Unreal Project - its feature set could be u  lized in games by merging (see Chapter 9). It is based on standard Unreal assets (Blueprints, Materials), does not contain C++ code / pre-compiled elements / third party content. None of the core func  ons uses Niagara. Cooking, compiling and packaging is tested for Windows PC, Apple PC, Android Mobile and iOS mobile.

Note1: Live blueprints are annotated, visually organized and cross referenced with links, to help developers understand and modify them. Example screenshots: BP1, BP2, BP3

Note2: while NinjaLive is currently implemented as a "Project", it could be turned into a "Plugin". See Chapter 13.

3. Loca  on & Size

The project is located in a single folder: /Content/FluidNinjaLive

Func  onal core is 35 Megabytes, Presets and Output Material Examples 15 MB, Tutorials 50

MB, the Shader and Asset Cache generated at first run is ~ 100 MB. The two most important

files, NinjaLive (Actor class) and NinjaLiveComponent (ActorComponent class) are 2 MB and

7 MB, respec  vely.

4. Versions

NinjaLive is developed in separate branches to ensure compa  bility with different UE versions. Check the version of your NinjaLive project at:  /Edit /Project Se   ngs /Project /Descrip  on

UE 4.23 - 4.25 branch

Latest NinjaLive version: 1.5.23.3

UE 4.26 - 4.27 branch

Latest NinjaLive version: 1.5.26.3

Featuring: Volumetric Clouds (clouds are not supported below UE 4.26)

UE 5.00 branch: officially does not exist (as of 21 Aug 2021)

UE5 is in the early access phase - and EPIC Marketplace does not allow developers to offer UE5 dedicated product versions. For this reason, UE 4.27 ninja branch has been tested under UE5. Test results: (A)  core func  ons running fine - (B) a cri  cal UE5 bug corrupts most setups

UE5 bug: GetPhysicsLinearVelocity and GetComponentVelocity blueprint nodes are failing. Nodes return ZERO for non-physics objects (eg. animated or blueprint driven objects).

The above two nodes work as expected under UE 4.20 - UE 4.27, returning correct velocity values for all kinds of moving objects. The bug has been reported to UE devs.

UE5 bug consequences: ninja massively u  lizes object velocity to drive/control simula  ons. In UE5 EA, objects leave density marks (as expected), but do not drag the fluid.

At stages where “kill brush at low velocity” feature is enabled: the stage is totally blacked out (nothing is visible or nothing happens - since UE5 constantly returns ZERO velocity).

Avoiding UE5 velocity bug: (A) “Single Target Mode” is immune, as it is using a different method to calculate velocity. (B) Texture and parameter defined velocity is also working. (C) By switching off the “kill brush at low velocity”feat, blackout stages could be fixed. See: NinjaLiveComponent /LiveBrushSe   ng /DampenBrushBelowThisVelocity ---> set to 0.0

5. Usage

NinjaLive is a standalone Project: open it in Unreal Editor, check the Tutorial Levels! When star  ng the game (pressing Play):

Use Selected Viewport Mode -- do NOT use Simulate Mode

For running ninja in Standalone Mode, see: chapter 21.3.6  (LiveCompa  bility) In order to u  lize fluidsim features in your project, NinjaLive should be merged:

6. Core Func  ons & Terminology

FluidNinja LIVE is created from scratch to complement FluidNinja VFX Tools - a sibling project, focusing on non-responsive / baked fluid simula  on. The two projects are shortly referred to as NinjaLive and NinjaTools. In this document, we are focusing exclusively on NinjaLive.

6.1 NinjaLiveComponent

FluidNinja Live is a library of func  ons built around a single, compact unit: "NinjaLiveComponent", o  en addressed as "simula  on component", located at

1.

2.

Prepare the target project to host NinjaLive, by se   ng the directly specified High Level Se   ngs - see Chapter 9 !!!

copy /Content/FluidNinjaLive to the /Content folder of the target project

/Content/FluidNinjaLive. It is an "ActorComponent" class object that should be added to a host object (eg. an "Actor" class object) in order to use it. The host is always called "owner". The sim component could run autonomously (loading its default preset, crea  ng RenderTargets for itself) - OR - it could connect to the interac  ve Preset Manager and to the

Once merging is done, place/spawn NinjaLive fluidsim Actors at your levels - or add NinjaLive fluidsim Component to your own object classes. To make fluidsim Component work properly in your Actors, apply the directly specified Low Level Se   ngs to the host actor class

( see Chapter 10.3 ).

Op  onally you could u  lize addi  onal tools like Preset Manager with custom GUI, Memory Manager, U  li  es and Interface Controller.

Fluidsim related parameters could be stored / loaded as presets and handled by the Preset Manager (See Chapters 6.4 and 21).

Func  onal parameters (eg. LOD se   ngs, interac  on filters) could be set at the Actor and ActorComponent Details panel - as exposed variables - or set by blueprints using Live Interface.

Besides NinjaLive Managers, UE Sequencer could be used to control SimContainers. See Chapter 23.

Memory Manager. An important restric  on: sim component does _not_ have built in overlap detec  on func  onality - and for this reason, interacts only with predefined owner components (eg. Bones, Sockets, Sta  cMeshComponents... ).

6.2 NinjaLive

"NinjaLive", an "Actor" class object, referred as "sim container" or "sim area", located at /Content/FluidNinjaLive ... is a dedicated owner, embedding "NinjaLiveComponent" and adding two important features to its core feature set:

(A) ac  va  on volume (usually much larger than the sim area): the proximity of any user defined agent could switch the sim component between wake/sleep states.

(B) interac  on volume (usually same size as the sim area): detec  ng overlapping / colliding objects and forwarding the spa  al informa  on + velocity to the sim component. Using the interac  on volume, anything could interact with the simula  on: Sta  c- and Dynamic Meshes, SkeletalMeshes, PhysicsBodies, Destruc  bles.

6.3 Actor Details VS ActorComponent Details

When selec  ng an Actor on level, the Details panel is displaying all the exposed variables of that instance. In the case of "NinjaLive" Actor, the variables are collected into 4 groups, all star  ng with "Live" prefix: Ac  va  on, Interac  on, Debug, ComponentOverrides.

IMPORTANT: NinjaLiveComponent se   ngs are exposed on the component level - accessible only at the Component Details panel - NOT visible at the Actor Details panel!

Accessing Component Details:

Select NinjaLive Actor - or any other sim component owner Actor on level. At the Details panel, there is a top sec  on, where Actor Components are listed (to see all components, this "top list window" should be rescaled). Select "NinjaLiveComponent".

Here is a visual guide on “how to select NinjaLiveComponent” --->	IMAGE LINK

Un  l the Component is selected, the Details panel displays its (the component's) proper  es - and NOT the owner proper  es. NinjaLiveComponent variables are collected to 9 groups, all star  ng with "Live" prefix:

Ac  va  on, MemoryManagement, Performance, Compa  bility, Debug, Generic, Interac  on, BrushSe   ngs, Raymarching

In this document, we are referring to the two states of the Details panel as "Actor Details" and "Component Details".

6.4 Preset Manager and Presets

Ninja simula  on parameters (eg. speed, forces, inputs) are stored in PRESET files. PRESET MANAGER is an editor only tool with a custom UI, made to…

A. develop / tweak fluidsim parameters in real-  me

B.  load / save se   ngs as preset files

PresMan. is a helper func  on, not needed to run ninja at all. In case you'd like to control sim via game logic like Preset Manager does, read Chapter 26.: "Controlling Live in real   me".

6.4.1  A level placed NinjaLive container is associated with a Default preset at: NinjaLive Actor Details /NinjaLiveComponent /LiveGeneric /DefaultPreset

Fluidsim params stored in the Default Preset are loaded when a sim container ini  alizes (No PresMan needed, containers are autonomous!)

6.4.2  Preset Manager could be used by placing  /FluidNinjaLive /NinjaLive_PresetManager blueprint actor on a level. The most important UI elements are (see more at 21.1):

A.  In the top row, you could enable Tool  ps (recommended!)

B.  In the second row (printed in red) you could SELECT a given NinjaLive Actor

C.  In the third row (printed in blue) you could LOAD a preset for the selected actor.

The default preset of a given NinjaLive Actor is marked with * symbol

D. Using the floppy disk icons, you could save and duplicate presets

6.4.3  More Preset Manager related topics in this Manual:

10.2.2	Preset Usage

14.1	Remove Preset Manager From Levels Before Compile

21.1	GUI exposed params

21.3.2	LiveGeneric /DefaultPreset

6.4.4   Preset Manager Error Handling

A. Level-placed PresMan could be de-ac  vated, at PresMan Actor Details Panel.

Icon turns gray (when enabled, it is black).

B.  PresMan tries to reach clients at startup. You could define the default client at the PresMan Actor Details panel. In case the default client field is empty / or client is not present / or client is asleep: PresMan tries to load someone else. Once started, you could manually select which client (which NinjaLive Actor) you would like to load for tweaking.

C.  Mul  ple Preset Managers per level: DO NOT

D. PresMan can't find clients - case1:

Preset Manager could go blank when it can not connect a valid ninja container. A possible cause: containers are usually set to be proximity ac  vated. When the pawn or spectator camera (that triggers the proximity sensor) is away, the container is inac  ve (asleep). PresMan can _NOT_ wake sleeping containers. This is communicated via an explicit on screen message.

E.  PresMan can't find clients - case2:

PresMan recognizes clients via the Live INTERFACE -- in case you are not using a the standard NinjaLive BP - eg. you have embedded NinjaLiveComponent to your custom BP, like a pawn, make sure you have added "Live Interface" under BP/Class Defaults/Interfaces

F.  PresMan can't find presets:

NinjaLive actors contain a non-dynamic reference for preset LOOKUP loca  on. By default, it is: /Content/FluidNinjaLive/Presets

In case you have moved ninja to a new subfolder: update the “PresetSearchPath” variable in NinjaLiveComponent Blueprint.

G. Mul  ple NinjaLiveComponents per Actor:

Preset Manager is currently not prepared to handle mul  ple NinjaLiveComponents embedded in a Single Actor. You could try developing presets for each component separately (using duplicated actors) - and when done, simply referencing the final preset files for each Component under LiveGeneric/DefaultPreset.

6.5 Simple Painter Mode: experimental feature

NinjaLive has the poten  al to track objects, detect collisions and write the data to a RenderTarget - this is what we call "Collision Painter". The func  on could also accumulate and erode paint strokes by   me.

As a (yet) experimental feature, we have built in a switch - SimplePainterMode - that (a) discouples Collision Painter from the rest of the system and (b) disables NinjaLive fluidsim func  ons. The result is a minimal system using only 1 RenderTarget, keeping GPU load on a very low level. Combined with the regular "User Editable Output Materials" feature, SimplePainter could be used efficiently for certain VFX types.

The switch is located at NinjaLiveComponent Details /Live General /SimplePainterMode (at the very bo  om)

Link: [See MODULE061 A-H in NinjaLiveComponent Blueprint]

Chart 1  Actors, Components: dependency and func  ons. High.res bitmap: LINK

7. Memory Demand

TYPICAL PERFORMANCE

LIVE	1.2  vs  1.3

GTX 1070, UE 4.23, average FPS

NinjaLive allocates memory mainly for RenderTargets. The rendering pipeline uses 8 RenderTargets for a single simula  on container by default: 1 four-channel [ RGBA, for CollisionPainter ], 5 single channel [ R, for Density and ScalarFields ] and 2 bi-channel [ RG, for VectorFields ] - all set to 16 bit precision. The pipeline could be reconfigured many ways (resolu  on, bit depth, channel usage).

A 256x256 fluidsim container with default se   ngs allocates 1664 Kilobytes of memory - this could be crunched or expanded, depending on the needs. Similarly: 512x512 area = 6656

Level 11: SmokeChamber test scene   FOUR 720 px  fluidsim containers Live 1.2: 140  FPS	vs	Live 1.3: 230  FPS

Level 13: SmokeChamber test scene   TWENTY 720 px fluidsim containers Live 1.2: 100  FPS	vs	Live 1.3: 190  FPS

Kbytes, 1024x1024 area = 26624 Kbytes, 2048x2048 area = 106496 Kbytes. Of course, the sim area could be non-square (eg. 128 x 512), mem consump  on is changing accordingly.

Link1: See Help.uasset /Chart2 /RenderTargets for more details on possible RT configs. Link2: Select any actor containing the sim component, and switch on "/Component Details /LiveDebug /ShowMemoryManagement" for dynamic, on-screen report on actual memory consump  on of a given container.

8. Performance (excerp  on)

Important: Live 1.3 has been released with major performance improvements. The new results are NOT YET included in Chapter 12 / the currently available data is outdated.

The following excep  ons (typical and peak) are here to represent the magnitude of changes.

PEAK PERFORMANCE

LIVE 1.3  (unlit scene, an  alias and postprocessing off) GTX 1070, UE 4.23

9. High Level Se   ngs: merging NinjaLive to your project Note: Live v1.1 merging process has changed - compared to v1.0.  Access legacy guide: LINK

Before merging, the target project needs to be prepared! NinjaLive requires the following Project Se   ngs to work properly:

STEP 1:  add custom Trace Channel

NinjaLiveComponent uses a custom channel - FluidTrace - to perform line tracing while projec  ng WorldSpace collision coordinates on the fluid simula  on mesh (TraceMesh).

A. Go to /Edit /Project Se   ngs /Collisions /Trace Channels

B.  Choose "Set New trace channel"

C.  Name = FluidTrace, Default Response = Ignore

Note 1 : Trace Channel Index does not ma  er - “FluidTrace” could be added to an exis  ng set of Trace Channels, no need to re-configure the order of channels. Note 2: As a consequence of the default "ignore" response, all objects will be transparent for NinjaLive line tracers - except dedicated TraceMeshes in NinjaLiveComponent owners.

STEP 2:  enable "UV from Hit" engine feature

Go to: /Edit /Project Se   ngs /Engine /Physics /Op  miza  on and enable the “Support UV From Hit Results” op  on. Collision data is projected on the TraceMesh by linetrace as "Hit". Using this feat, 3D Hit coords could be converted to 2D sim UV-space instantly.

Please double check the above steps - compare your editor to this screenshot: LINK

STEP 3:  merging

A. Quit Unreal. Copy "/Content/FluidNinjaLive" subfolder from the original (non-merged) Live project to the target project root (/Content) using Windows or iOS file manager, while UE is not running.

Emphasis: do not copy the whole NinjaLive project structure to the target project - copy only the “FluidNinjaLive” subfolder located in the “Contents”. Do not try to use Unreal Migrate or Import func  ons - copy the above described subfolder.

B.  Open target project in Unreal Editor. Open NinjaLive Blueprint with blueprint editor.

Loca  on: /Content/FluidNinjaLive/NinjaLive.uasset Press “Compile”, then “Save”.

C.  Open NinjaLiveComponent Blueprint with blueprint editor.

Loca  on: /Content/FluidNinjaLive/NinjaLiveComponent.uasset

Set the default value of “TraceChannel” and “CollisionChannel” variables to “FluidTrace” See this visual guide on blueprint edi  ng: LINK

Press “Compile”, then “Save”.

In case the blueprint compiles without errors, we are almost done.

D. In the level editor, select any NinjaLive Actor on a level. Select NinjaLiveComponent

(see HOW). Check the “Live Compa  bility” group: the top 3 input fields should be set to “FluidTrace”.

Once done with these steps and checking: PROJECT MERGING IS FINISHED

10. Low Level Se

ngs

10.1.1 DISABLE/ENABLE

NinjaLive actors could be "force deac  vated" using the "Disable" flag (a) Select one/more "NinjaLive" Actor on level

(b) find "LiveAc  va  on" param group on the Details panel

10.1 Ninjalive Actor Parameters (briefly)

Chapter 21.2 is providing a full descrip  on on all NinjaLive Actor parameters. Right here, we are covering only a few key se   ngs.

Once the original NinjaLive project is opened - OR - merging NinjaLive to another project is finished, NinjaLive Actor (sim container) is ready to be deployed to game levels.

(1) drag-n-drop NinjaLive from the Content Browser to a level - OR - (2) copy-paste already configured Actors from NinjaLive tutorial levels to your project levels - and modify them to get sa  sfactory results. Start experimen  ng with the se   ngs, try to understand how Actors on tutorial levels are set up. You can find tutorial videos in this playlist.

When selec  ng a "NinjaLive" Actor on level, the Details panel is displaying exposed variables, collected in FOUR groups

LiveAc  va  on: [9 params], disable switch + wake/sleep related params (ac  vator, range...etc) LiveInterac  on: [12 params], scale sim area + filter interac  on types by class or bone name LiveDebug: enable on screen status messages

LiveComponentOverrides: helper func  on to set variables when mul  ple actors are selected.

See NinjaLive blueprint / GROUP1 / SUBGROUP003 for more informa  on.

(c) set Disable flag state: on/off

The disable switch OVERRIDES ac  va  on se   ngs: the Actor remains passive even if the ac  vator is in range. Once disabled, the on-level Actor ICON turns gray (originally red).

10.1.2 SCALING / SIZE

Do _not_ try to scale the sim container actor via viewport transform gizmo - nor using the numeric Scale Transform input at the Actor's Details panel. Find the "Live Interac  on" param group at the Actor's Details panel, and set "Trace Mesh Size" and "Interac  on Volume Size" to influence the size of fluid simula  on plane and the size of interac  on volume, respec  vely.

The default TraceMesh, found in NinjaLive Actor, is a planar mesh. For this reason, when you set "Trace Mesh Size" on the details panel, the Z-component of the input field does not have any effect on the visuals and workings. Later on, you will learn how to add a custom TraceMesh to your own actors. In case these meshes are not planar (eg. a hemisphere), the Z-scaling component starts to make sense - and that is the reason for that param being there.

The Interac  on Volume is a true 3D object by default, so all dimensions of the "Interac  on Volume Size" param field have effect on the workings.

Note: TraceMesh and Interac  onVolume both have a separate visibility flag (at "Live

Interac  on" group). Make sure that these flags are on - to visualise the men  oned geometries. The two params are: "ShowTraceMeshInEditor" and "ShowInterac  onVolumeInEditor"

10.1.3 NON-UNIFORM SCALING

Using the above men  oned op  ons, you could set up a rectangular (non-square) TraceMesh/ simula  on plane. Since fluidsim XY resolu  on is also manually provided (non-automa  c), you should make sure that side-propor  ons match - eg. a TraceMesh, sized X=4,Y=8 is matched with a Fluidsim resolu  on set to 256x512 or 512x1024.

Note: you could reach fluidsim resolu  on se   ngs at the Details panel of NinjaLiveComponent. Select NinjaLive Actor. At the Details panel, there is a top sec  on, where Actor Components are listed (some  mes, to see all components, this "top list window" should be rescaled - by dragging the bo  om part of the window-handle downwards). Select "NinjaLiveComponent". Un  l the component is selected, the Details panel displays its (the component's) proper  es - and NOT the owner proper  es.

Find "Live Performance" submenu, set "Resolu  on X/Y" respec  vely.

10.1.4 POSITION & ROTATION

As opposed to Scaling: Posi  on and Rota  on of the sim area should be set using the viewport gizmo or the Transform input field at the Actor's Details panel.

10.1.5 LOCALISING INTERACTIONS: OVERLAP PROJECTION

Params related to this sec  on are located at: Actor Details /Live Interac  on

While our simula  on is 2D, we would like to seamlessly integrate it to 3D space - so, we came up with the "Overlap Projec  on" technique. Our fluid simula  on container actor, placed on level has two related components:

(A) Interac  onVolume: used to detect overlapping / collision - objects inside this volume could interact with the simula  on - ini  al and ongoing overlap events are con  nuously monitored (both entering and leaving the volume). The type of interac  ons could be defined / filtered by the user (see next chapter)

(B) TraceMesh: we are mapping the simula  on output to this mesh (by default a plane, but could be defined by the user) plus: we are going to perform a line trace against this mesh.

The sim container is triggered by objects that overlap with the Interac  on Volume. Once overlapping object(s) / components are targeted, a line-trace is performed in their direc  on (A) from the camera or (B) from a user-defined point - against the TraceMesh. Line-trace hits the TraceMesh, the hit informa  on is transformed to TraceMesh UV space (which is also simula  on UV space), and the simula  on density/velocity inputs are feeded with the related posi  onal/velocity/size data.

Velocity could be derived from Frame/PreviousFrame posi  onal delta or alterna  vely by calling (reques  ng from the engine) the world space velocity of the colliding object and transforming it to sim space. NinjaLive uses both methods. What if the mo  on is perpendicular to the TraceMesh? NinjaLive contains a certain processor to handle this, by increasing pressure and velocity noise at the given loca  on, giving an illusion of an object "blas  ng through" a surface, producing shockwaves (if velo is large).

Non-perpendicular velocity vectors cause "drag-like" advec  on. We always check composite velocity, and add "blast factor" propor  onally to the perpendicular component.

Now, imagine that we are forcing TraceMesh to be camera facing. So we are mapping collisions from the camera view to a camera facing plane. And remember: we are tracking objects / bones that are inside Interac  onVolume, but not aligned on TraceMesh surface (imagine the limbs of a character or the debris of a crumbling wall): since trace-targets are moving in 3D space and the advec  on / density they generate is always posi  onally following them, the fluid seems spa  ally extended.

More on Camera Facing: 10.2.5

10.2 Ninjalive Actor Component Parameters (briefly)

10.1.6 FILTERING INTERACTIONS

Params related to this sec  on are located at: Actor Details /Live Interac  on - allowing you to specify / filter interac  on types for the simula  on area.

/Overlap Filter Inclusive Object Type: an array, where you could add/remove object type-classes: WorldSta  c, WorldDynamic, Pawn, Vehicle, Destruc  ble, PhysicsBody /Auto Exclude Large Overlapping Objects: eg. an extended floor object that overlaps the

Chapter 21.3 is providing a full descrip  on on all NinjaLive Component parameters. Right here, we are covering only a few key se   ngs.

Select any NinjaLive-Component-Owner Actor on level (eg. "NinjaLive" Actor or Orb, Pawn). Select NinjaLiveComponent* at the Actor Details panel (see Actor Components list at the top)

Interac  on Volume, and we do not want it to interact with the fluid. You could do the same manually , under:

*Here is a visual guide on “how to select NinjaLiveComponent” --->

IMAGE LINK

/Exclude Specific Actors From Overlap

Bone names: in case you have provided a skeletal mesh class (eg. Pawn) in the Filter array, the container is trying to detect all bones by default - most of the cases this is not what we want. "Par  al name filter" allows you to type in "foot" ot "hand" - and all bones containing these strings will be included. Performance-wise, the most op  mal solu  on is providing "Exact Bone Names": eg. "feet_r", "feet_l" for a Mannequin, walking in a puddle sim.

10.1.7 WORLD SPACE VELOCITY

There is a single param for channelling World Space velocity to the simula  on: eg. we have a ke  le full of swirling liquid, and we start to push the ke  le / or we have a pawn with flaming hands, and he starts to run, and we expect the smoke/flames to be le   behind.

The param that influences this phenomena is a "preset" param, that could be changed using the preset manager,

see "Velocity Field influenced by sim area mo  on" on the GUI - or modified by manually edi  ng the preset file (defined at Component Details / LiveGeneric) at

No  ce: as you select the actor component, the "Details" panel changes and you could see the exposed Component Variables, collected in 9 groups, all star  ng with "Live" prefix.

Listed in order of importance:

Interac  on [11 params] - con  nuous (non-overlap based) interac  on AND single target mode Generic [14 params] - PRESET and INPUT / OUTPUT MATERIAL defini  ons + more Performance [19 params] - resolu  on, LOD, render pipeline se   ngs

MemoryManagement [3 params] - in case mem.manager is placed on level: set up connec  on Raymarching [16 params] - picking lightsource for raymarching, se   ng params

Compa  bility [6 params] - system level switches, eg.: Flip RenderTargets for Mobile compile BrushSe   ngs [11 params] - collision painter brush size overrides and noise se   ngs

Ac  va  on [1 param] - component level disable switch (actor level is preferred, when available) Debug [10 params] - enable on-screen repor  ng of LOD, memory...etc

10.2.1 DISABLE/ENABLE

When talking about NinjaLive Actor, we have TWO disable switches available:

"VeloFromSimAreaMo  on" field.	Actor Details /LiveAc  va  on /Disable	(preferred)

Actor Component Details /LiveAc  va  on /Disable	(not preferred)

Normally, we disable systems on the Actor Detail level. Once NinjaLiveComponent is added to a random owner (eg. pawn), the high level (actor level) disable is not there anymore. In this case, the component level disable could be used.

10.2.2 PRESET USAGE

Component Details /Live Generic /Default Preset - this is where you could associate a default preset for a NinjaLiveComponent - this preset will be automa  cally loaded at area

ini  aliza  on. Note: in the Preset Manager, you could load / try arbitrary presets for a given area - but you have to provide DEFAULT here, at the Component Details.

10.2.3 OUTPUT MATERIAL USAGE

Output Materials: NinjaLive rendering pipeline is a chain of Blueprint controlled Materials,

wri  ng to RenderTargets. Any RenderTarget could be used as output, depending on the needs (typically Density, Velocity, Pressure and PainterInput). So, a typical "Ninja Output" material contains at least one "Parametric Texture Object" with Specified name, eg. "DensityBuffer", "PressureBuffer"...etc. these materials are tapped into the rendering pipeline, and use the needed raw sim output to create arbitrary shaders.

Note: by default, the render pipeline is processing MONOCHROMATIC density data, and the output is COLORIZED - based on the available data (eg. Tonemapping monochrome density with two colors). The density could be sent to a raymarch shader or Pressure used to perform WorldPosi  onOffset, Divergence for Refrac  on... really up to you. Have a look at the provided output material examples at: /Content/FluidNinjaLive/OutputMaterials/

Below the array of output materials, there is an INTEGER input field, labeled as:

“Output Material Selected” - provide here the array index for your default/favourite material. The array is also accessible in the Preset Manager - so you could check varia  ons in run  me.

Btw, run  me: you could start/leave open the standard Unreal Material Instance Editor UI and tweak output material params while Ninja is running - and you see the real   me fluidsim in the main viewport - very efficient way to fine-tune params!

10.2.4 RAYMARCHING

To set up NinjaLiveComponent for Raymarching, check these se   ngs at Component Details: ●  LiveRayMarching / Enable Raymarching switch

●  LiveGeneric / OutputMaterials array: make sure there is a raymarch capable output

material on the list

●  LiveGeneric / OutputMaterialSelected: set the array index for the above material

10.2.5 INTERACTIONS

Component Details /Live Interac  on: these se   ngs are partly described in the next chapter, see: STEP 9-10, Se   ng up "non-overlap based" or "con  nuous" interac  on with the owner.

Note: Con  nuous Interac  on should be flagged ON every   me you include Live Component to a new owner (the new host is NOT NinjaLive Actor, but some other class) - this flag ensures that LiveComponent could interact with the new owner. Other se   ngs in this submenu:

Single Target Mode: focusing on a single superfast target (eg. pawn fist)

Character anima  on could be erra  c, extremely fast movements occur, the temporal sampling of the collision painter simply can not cope with this, we see density dots. This mode focuses on a single target, and connects the sampling points via interpola  on, crea  ng a con  nuous mo  on-trajectory, drawing density and velocity lines between the sparse sampled posi  on data points - this way making "fluid ribbons" / "smoke or flame trails" behind the superfast moving object. The target is the first item in the "Bone names'' list - or the first element found based on type filtering.

Using the "Single Target Set Sim Speed" flag, a bullet-  me like "  me dila  on effect" could be achieved: the moving speed of the single target influences fluidsim play speed.

Camera Facing: performed by a generic func  on, stored at: /Content/FluidNinjaLive/Core/NinjaLiveFunc  ons - usually called by the sim component (see MODULE036 in the blueprint)

Important Note: when using NinjaLive as a component IN MOVING AGENTS (eg.: a Pawn),

keep Camera Facing in the sim component switched OFF and call the "Camera Facing" func  on in the owner Blueprint's Tick flow. This results in smoother rota  on. Reason: the   cking of Pawn transform and TraceMesh transform should be synchronized (in case of non-moving owner sync does not ma  er).

See included "mo  le owner" type blueprints - referred at 10.3

10.2.6	USE CUSTOM LINE TRACE SOURCE	(instead of camera)

New feature, added to NinjaLive 1.2 ---- UI access:

NinjaLiveComponent Details Panel /LiveInterac  on /UseCustomTraceSource NinjaLiveComponent Details Panel /LiveDebug /VisualizeCustomTraceSource

Levels demonstra  ng the feature: LEVEL 2C -- LEVEL 21 Stage 6B -- LEVEL 24A, 25, 26 Tutorial video on feature: LINK

NinjaLive uses line tracing to project 3D collision data to the 2D simula  on plane. By default, the trace line starts from the camera and targets the overlapping / colliding object. At the point where the trace line intersects the TraceMesh, the collision painter is drawing to the collision buffer. This is how overlapping objects generate density/velocity for the simula  on.

Using the new feature, we could define a custom line tracing source point, by adding XYZ offset to the sim container posi  on.

Con  nuing 10.2.6

Advantage 1: no visual artefacts when looking at planar surfaces from low angle ("FPS view")

Advantage 2: detaching simula  on container and simula  on output - running the sim in the background while the user and the camera is somewhere else

Note1: before adding this new feature, NinjaLive was already capable to run "non-interac  ve" simula  ons in the absence of camera, eg. using a texture or material to generate density while ignoring overlapping objects --- see Level 24B setup!

Note2: to perform the "run a sim while we are away" feature, we need to switch off LOD and "Pause sim when not visible" --- see Levels 24A,24B,25,26 for details!

Advantage 3: no impact doubling when running the simula  on on hollow meshes and viewing "from the other side". Imagine a sphere, used as TraceMesh - and an object colliding to the RIGHT hemisphere. In case we watch the impact "from the other side" the trace line (star  ng from the camera) crosses the LEFT hemisphere as well, genera  ng UNWANTED simula  on input. In case we are NOT using the camera as trace source, this could be avoided --- see Level21, Stage 6B!

Advantage 4: we could avoid VR artefacts - when the user is rapidly changing his point of view and the overlapping object "hovers" above the TraceMesh (not exactly intersec  ng the tracemesh), the projected collision point wanders (like a shadow of an "above the floor object" while you are moving the torch). By using a fixed line trace source point, this could be avoided.

10.3 Adding NinjaLive Component To Your Own Actor Classes

We are going to update an exis  ng actor class to contain NinjaLiveComponent. Typical cases include Pawns, Vehicles and other mo  le agents.

Before start, we'd like to men  on: on the NinjaLive tutorial levels you'll find mul  ple examples for this setup - including a floa  ng orb and the classic UE-Mannequin pawn, both with NinjaLiveComponent added. Blueprints could be found at these loca  ons (folders):

/FluidNinjaLive/Tutorial/BP_NinjaDemo_MovingBall1_NinjaComponentAdded

/FluidNinjaLive/Tutorial/UE_Mannequin_UsageExamples/ThirdPersonCharacter_NinjaAsComponent

Important: NinjaLiveComponent does _not_ have built in overlap detec  on func  onality, interacts only with predefined (user defined) owner components (eg. Bones, Sockets, Meshes). This type of interac  on is labeled "non-overlap based" or "con  nuous" interac  on, handled by MODULE021 in the NinjaLiveComponent blueprint. Below, you'll find instruc  ons on how to manage this (STEP 8-9, explained)

Let us start with a quick step-by-step guide, followed by explana  on on the non-trivial steps:

1.  open owner (the planned sim component host blueprint)

2.  add component: NinjaLiveComponent

3.  add component: TraceMesh (Sta  cMeshComponent)

4.  set the value of "TraceMeshComponent" variable of NinjaLiveComponent (value =

TraceMesh)

5.  set TraceMesh proper  es (note: scale X,Y is related to sim X,Y resolu  on!)

6.  add NinjaLiveInterface to the owner blueprint (at Class Se   ngs /Interface)

7.  set NinjaLiveComponent default proper  es: while remaining in the blueprint editor, select NinjaLiveComponent in the "Components" list of owner to access NinjaLiveComponent Details panel. Find "Live Interac  on" parameter group

8.  set the "Con  nuous Interac  on with Owner Actor" switch to "ON"

9.  set con  nuous Interac  on Inclusive Object Type / Bone names

10. set the default value of all other params here (in the blueprint) - eg. default preset,

material

11. enable TraceMesh CameraFacing (op  onal)

12. save blueprint, and drag it on level from the Content Browser

STEP 3, ADD TRACEMESH COMPONENT TO OWNER

All NinjaLiveComponent owners must contain a Sta  cMeshComponent labeled as "TraceMesh". TraceMesh has two func  ons: linetrace is performed against it - and the fluidsim is mapped on it. Depending on the context, we could call this mesh "simula  on plane" or "tracemesh" - it is the same.

Any mesh would do - NinjaLive contains a simple Sta  cMesh type plane as default object, add that one for start: /FluidNinjaLive/NinjaLiveTraceMesh

STEP 4, TELL NINJALIVE ABOUT THE TRACEMESH

NinjaLiveComponent contains a "TraceMeshComponent" variable that MUST be set by the owner. Please have a look at this blueprint:

/Content/FluidNinjaLive/Tutorial/BP_NinjaDemo_MovingBall1_NinjaComponentAdded

OnEventBeginplay, set TraceMesh as "value" to "TraceMeshComponent'' variable of

NinjaLiveComponent. Steps:

a. pull NinjaLiveComponent to the blueprint edi  ng-surface

b. pull a data-flow thread from the component, and type: "TraceMeshComponent", place a "set" node

c. pull TraceMesh  to the blueprint edi  ng-surface - and wire it to the above "SET" node.

This way, we are explicitly telling NinjaLiveComponent about the TraceMesh, contained by the owner. From now on, the component does a lot of things with the mesh, all automa  cally.

STEP 5, SET TRACEMESH PROPERTIES

a. Editor Visibility could be switched off: TraceMesh component details /Rendering / Visibility

b. Collision se   ngs are extremely important - but NinjaLiveComponent handles this automa  cally by force se   ng all the necessary params: see MODULE004 in the sim component.

One thing that could _not_ be set from blueprint: double sided. This property should be enabled at the Mesh Editor. For our ready-to-use plane it is flagged "on''.

c. Scale: three methods to set TraceMesh scale

- At the level placed instance - by selec  ng the component (this effects only that instance)

- In the owner blueprint, selec  ng TraceMesh and set default transform (effects all instances) - Via an exposed variable (NinjaLive Actor does it this way, so you could customize instances)

Important: scale is related to simula  on X,Y resolu  on - see: 10.1.3 NON-UNIFORM SCALING

STEP 6, ADD INTERFACE

Add NinjaLiveInterface to the owner blueprint at: Class Se   ngs / Interface

NinjaLiveComponent-Containing-Actors (owners) should have this interface implemented...

a. to be visible/accessible for GUI-controlled preset management

b. to be excluded from the line tracing of OTHER owners

c. to access / modify exposed variables of any level placed sim component owner

Both NinjaLiveComponent and NinjaLive_PresetManager iden  fy / collect "clients" based on INTERFACE (Get all Actors with Interface).

See: NinjaLiveComponent: MODULE020 (exclusion from linetrace) / NinjaLive_PresetManager: SUBGROUP003 On Tutorial Level 5 you could find a setup where NinjaLive_InterfaceController is communica  ng with a NinjaLive Actor using the interface, ini  a  ng a "fade off / shutdown" sequence, have a look! The clients are defined in the Actor Details/ Interface Target Actors array of controller.

STEP 8-9, SETTING UP "NON-OVERLAP BASED" OR "CONTINUOUS" INTERACTION WITH THE OWNER

Next, we are going to set NinjaLiveComponent default proper  es. While remaining in the blueprint editor, select NinjaLiveComponent in the "Components" list of owners - to access NinjaLiveComponent Details panel.

At the Component Details panel, find the "Live Interac  on" parameter group. Set the "Con  nuous Interac  on with Owner Actor" flag to "ON".

When a sim component is embedded to its na  ve host - NinjaLive Actor - the owner is doing overlap detec  on and telling the component's line tracer "who to track". Sim component does not have built-in overlap detec  on func  onality. When embedded to new owner (eg. pawn), line tracer falls back to a user defined list on "who to track" - the list contains type filters

(Sta  cMesh, Dynamesh, Pawn...etc) and bone/socket lists ("Inclusive Object Type" and "Bone names" ). This is all very similar to the Interac  on se   ngs of the Actor Detail menu - see

10.1.6 FILTERING INTERACTIONS for a general descrip  on on interac  on filtering and bone names.

STEP 10:

Set the default value of all other params - eg. default preset, material We have been talking about this in the previous chapter. See:

10.2.2 PRESET USAGE, 10.2.3 OUTPUT MATERIAL USAGE

While the brush size can be adjusted via Preset, you may want to tweak it in: Component Details /LiveBrushSe   ngs

STEP 11: (OPTIONALLY) ENABLE TRACEMESH CAMERA FACING at: Component Details /LiveInterac  on

Please note: with rota  ng/mo  le agents, it is be  er to call the Camera Facing func  on in the owner blueprint, see 10.2.5 INTERACTIONS / Camera Facing for case descrip  on.

10.4 Other Ways To Include/embed/add Ninjalive To Actors

Adding NinjaLiveComponent to an object class is the preferred / advised way to equip any agent/object with fluidsim. However, there are other methods to do this. The updated version of this document will feature a chapter describing these methods. Briefly:

A. CHILD ACTOR COMPONENT

Unreal Engine implemented a structure not too long ago, called "ChildActorComponent". The structure enables us to add "Actor" class objects as components to another class (as opposed to adding "ActorComponent" class components). The UE implementa  on looks somewhat unfinished and accessing child parameters is a bit confusing - but we can add NinjaLive Actor with all it's overlap and proxy ac  va  on management systems to any actor. We have managed to successfully embed NinjaLive Actor to the default Mannequin Pawn, see: ThirdPersonCharacter_NinjaAsChildActorComponent in this folder:

/Content/FluidNinjaLive /Tutorial /UE_Mannequin_UsageExamples

B. SPAWNING

NinjaLive Actor could be spawned by code / blueprints - and a  ached to the spawner, crea  ng a similar situa  on as if it was an "owner". The main problem is, how to (pre)define the params of the "to-be-spawned" Actor? Two ways to do this:

(a) spawn, set params instantly, re-ini  alize the spawned Live actor

(b) by flagging "Expose on spawn" ON for variables in the "to be spawned" Blueprint (in this case NinjaLive), we could access them / set them before spawning - on the spawning node. We have tested this as well - see: ThirdPersonCharacter_NinjaSpawnedA  ached in this folder: /Content/FluidNinjaLive /Tutorial /UE_Mannequin_UsageExamples

10.5 Interac  on: se   ng up Objects to trigger NinjaLive response

---> See Level 21 / Stage2

NinjaLive is pre-configured to handle a wide range of interac  ons as "recipient" - but some steps should be taken on the "causer" side as well (causer = collider, overlapping object).

A. OVERLAP

In case you want an agent to trigger NinjaLive Actor's OVERLAP sensor:

ck ON the "Generate Overlap Events" op  on at Details /Collision (by default, it is OFF) Repea  ng: by default, all build-in Unreal objects (eg. a sphere, a plane) are set "not-to" generate "overlap events", this se   ng should be changed.

An interes  ng fact [UE bug? :) ] - in case an object is INITIALLY (at BeginPlay) in the overlap zone (inside the interac  on volume) of NinjaLive Actor, it will be detected as overlapping agent - the “generate overlap” se   ng influences only "latecomers" and everybody who wants to "leave". In case you have an agent that (1) starts in the area and (2) later leaves and (3) "GenerateOverlapEvents" is NOT enabled - NinjaLive will not receive a no  ce on the leaving and this might lead to anomalies.

B. SKELETAL MESHES

NinjaLive starts to track SkeletalMeshes if the "Pawn" interac  on filter is ac  ve/added. What if a SkeletalMesh is not a pawn? Eg.: you place a SkeletalMesh class object to level and set it to play an anima  on, and it is not embedded in a blueprint? Not a problem: set the SkeletalMesh (or the object that contains it) to interact like a pawn:

Go Details /Collision and set "Collision Preset" to “Pawn”

In case you do not want to modify the whole collision preset, try to set only the “Object Type”.

11. U  li  es: Possess Nearest Pawn / Quality Se   ngs

NinjaLive_U  li  es Blueprint Actor could be op  onally placed on game levels to help development. Symbol: a green le  er N. Main features:

1.  LIGHT WIDGET

Add main (direc  onal) light rota  on and intensity controller widget to viewport To use this feat, you need to manually add the widget and the target light.

2.  EDITOR FPS

By default, UE is set to run at 120 FPS. As a simple kind of performance measurement, legacy ninja versions (un  l 1.3) were UNLOCKING the UE limit, using the following console command: t.MaxFPS 900 (900 FPS is an arbitrary, theore  cal limit)

As a result, tutorial levels were running with no FPS-limit - usually around 200-400 FPS on a GTX 1070. This was a simple, good way to have instant feedback on performance bo  lenecks. With the release of RTX 3000 series GeForce cards, a problem emerged: these cards were able to run test levels on 900+ FPS --- and the card got exhausted. The “Override Editor 120 FPS limit” got switched off, by default (on most levels).

3.  POSSESSING PAWN

Possess Nearest Pawn is a very useful func  on: In case it is OFF, we have a free / unbound spectator camera while in Play. In case it is ON, it will find the NEAREST pawn (compared to our current posi  on in the world) and possess that. Ninja tutorial levels are o  en populated with mul  ple pawns at different stages - it is handy to navigate in editor, press Play and instantly possess that given pawn at that given stage.

4.  The u  l (once placed on level) influences other SYSTEMIC things, like DOF, mo  on blur,

an  alias, camera and mouse-cursor smoothing.

12. Performance Tests

Important: on 2 June 2021 Live 1.3 was released with major performance improvements. The new results are NOT YET included in this chapter / the currently available data is outdated.

The excep  ons in CHAPTER 8 are here to represent the magnitude of changes. You could read more about performance improvements in the ChangeLog (LINK)

12.1 Tes  ng environment

Hardware: Nvidia GeForce GTX 1070, 1920x1080 HD resolu  on

So  ware: Win10 64bit, DirectX 12, UE 4.20, ac  ve viewport switched to fullscreen, zoomed in - area of interest fills the viewport

Editor FPS cap raised (default=120) - using console command: "t.MaxFPS 999" Simula  on FPS limited is to 120 (no more than 120 frames calculated, per second)

12.2 Peak performance

Test 01 mod (non-full-screen, excep  on)

Single, 512x512 sim container with single collider, default sim quality Translucent, unlit material

As an excep  on (compared to other tests) viewport is zoomed out, FX consumes only 25% of screen space. Average cost: 1.5 millisec per frame - viewport FPS fluctua  ng around 670 FPS

Test 01

Single, 256x256 sim container with single collider, default sim quality Opaque, unlit material.

Average cost: 1.7 millisec per frame - editor viewport FPS fluctua  ng around 590 FPS

Test 02

Single, 1024x1024 sim container with single collider, default sim quality Opaque, unlit material

Average cost: 1.9 millisec per frame - editor viewport FPS fluctua  ng around 526 FPS

Test 03

Single, 4096x4096 sim container with single collider, default sim quality Opaque, unlit material

Average cost: 8.6 millisec per frame - editor viewport FPS fluctua  ng around 116 FPS

12.3 Peak performance with Raymarch shader

Test 04

Single, 256x256 sim container with single collider, default sim quality

Translucent material with raymarch shader, processing data from 1 local lightsource Average cost: 2.1 millisec per frame - editor viewport FPS fluctua  ng around 476 FPS

Test 05

Single, 1024x1024 sim container with single collider, default sim quality

Translucent material with raymarch shader, processing data from 1 local lightsource Average cost: 2.3 millisec per frame - editor viewport FPS fluctua  ng around 434 FPS

Test 06

Single, 1024x1024 sim container with 10 collider Colliders: spawned, circular moving objects

Translucent material with raymarch shader, processing data from 1 local lightsource Average cost: 2.5 millisec per frame - editor viewport FPS fluctua  ng around 400 FPS

12.4 Collider stress

Test 07

Single, 256x256 sim container with 10 collider Colliders: spawned, circular moving objects Default sim quality, Opaque, unlit material

Average cost: 1.9 millisec per frame - editor viewport FPS fluctua  ng around 526 FPS

Test 08

Single, 256x256 sim container with 128 collider

Collider: Destruc  ble mesh falling apart to chunks, each piece lands and remains ac  ve in the sim area

Default sim quality, Opaque, unlit material

Average cost: 7.2 millisec per frame - editor viewport FPS fluctua  ng around 138 FPS

12.5 Combined test: proximity ac  va  on / memory usage / medium stress

Test 09

Level configura  on: 256 sim containers arranged in a 4 x 64 grid, ac  vator agent moves along the array, sim containers gradually wake / go to sleep as the ac  vator passes. On wake/sleep, they acquire/release RenderTargets. Ac  va  on range is set to guarantee: on average, we have

16 ac  ve sim areas at a given   me.

Sim container configura  on: 256x256, one collider per container, default sim quality, opaque unlit material

Camera follows the ac  vator agent, looking down / slightly towards the horizon, all containers visible. Average cost: 7.0 millisec per frame - editor viewport FPS fluctua  ng around 140 FPS Peak memory consump  on: 30.8 Mbytes

12.6 Maximum stress, test with LOD

Level configura  on: 64 sim containers arranged in a 8 x 8 grid, all ac  ve, running simultaneously

Container configura  on: same setup as with peak performance test (256x256)

LOD se   ngs: (1) pause off-screen sim areas, (2) distance based sim FPS reduc  on, (3) distance based pressure itera  on cycle number reduc  on

Setup1: camera is close to the grid, 14 containers visible (distance based LOD inac  ve) Average cost: 8.2 millisec per frame - editor viewport FPS fluctua  ng around 120 FPS

Setup2: camera zoomed out, 64 containers visible

Average cost: 18 millisec per frame - editor viewport FPS fluctua  ng around 55 FPS

12.7 Maximum stress, test without LOD

Level and sim configura  on is similar to the above

Distance based LOD features are turned off. "Pause off-screen sim areas" is on.

Setup1: camera is close to the grid, 14 containers visible

Average cost: 8.2 millisec per frame - editor viewport FPS fluctua  ng around 120 FPS

Setup2: camera zoomed out, 64 containers visible

Average cost: 74 millisec per frame - editor viewport FPS fluctua  ng around 14 FPS

12.8 Performance measurements under UE 4.23 - 4.26

In general, higher UE versions tend to produce lower performance.

At the extremes, the difference is significant. In the typical / recommended usage cases: not too bad.

A. EXTREMES

Peak performance: an empty, fullscreen UE 4.20 editor viewport could run 700 FPS once FPS cap is removed - and it drops to 590 FPS during peak performance test. For UE 4.23 the maximum number for an empty viewport seems 400 FPS, drops to 300 during peak performance tests. Consequently, the measurable performance of an empty UE 4.23 viewport is worse than 4.20 with Ninja peak performance tests running.

Stress: running 16+ ac  ve containers under 4.23+ got much worse - 30-50% loss, compared to UE 4.20 --- Performance drop cause: the handling/batching of RenderTargets has changed significantly in UE 4.23+

B. TYPICAL USAGE

Apart from the extremes, tes  ng in the typical / recommended usage zone:

Hi profile single container (HD, raymarch.) performance is 5-10% worse, compared to UE 4.20 Running 4-16 containers parallel, performance is 10-25% worse, compared to UE 4.20

Conclusion: despite these losses, we are s  ll good in the recommended usage zone.

13. Turning Ninjalive To A Plugin

NinjaLive is currently implemented as a "Project" - but it could be turned into a "Plugin". Charles Cline was kind enough to create a detailed guide for conver  ng NinjaTools to a plugin (back in July 2020): the same method could be applied to NinjaLive.

Here, we are linking the original PDF:

h  ps://drive.google.com/file/d/1OugHbFj9mI3gwe8Mk1RbNQ3wsARBB8qS Thank you very much for the knowhow, Charlie!

14. Packaging / Compiling in general

14.1 Remove Preset Manager From Levels Before Compile

(A) Preset Manager is a developer feature: it is not needed for the final product to func  on (B) PresetManager is using "Editor Only" func  ons (for file management) that could block compiling - it is advised to remove it from Game Levels completely once development is finished / before compiling

14.2 Please carefully study chapter 21.3.6 Live Compa  bility in this document

Compa  bility se   ngs include Trace Channel management and Dynamic Texture Lookup management - both being cri  cal / having an impact in project compiling / packaging.

(A) trace channel “autofind” does NOT work in packaged projects - users need to define trace and collision channels manually before packaging - this is a global se   ng (could be forced to all containers by defining trace channel variable values in the NinjaLiveComponent blueprint).

(B) dynamic texture lookup does NOT work in packaged projects

●  METHOD1: define sim density and velocity input textures on a per container basis

at NinjaLiveComponent /LiveCompa  bility /Overwrite*Input (NOT a global se   ng)

●  METHOD2: go to Project se   ngs /Packaging /Addi  onal Non-Asset Directories to Package and link the folder where the velocity and density maps are, this forces the system to package the maps. (GLOBAL se   ng)

14.3  NinjaLive 1.1 - 1.2 blueprints are branched to APEX and non-APEX versions

In case you are not using the APEX destruc  on plugin, you could completely remove all APEX related assets from the project before compiling. APEX assets are located at:

/Content /FluidNinjaLive /Versions

You could read more about the APEX removal in the Changelog: LINK (see v1.1 changes) Important: in NinjaLive 1.3 and higher versions, all APEX related content is removed.

14.4 Mobile Rendering, Limita  ons

(a) Android graphics drivers are flipping RenderTargets ver  cally. Since Ninja is using a chain of RenderTargets, only ODD pieces (seem to) flip! To prepare the system for this, go to: NinjaLiveComponent Details /LiveCompa  bility - and set "Flip RenderTargets for Mobile" to

ON (before compiling).

(b) Parallax Mapping is not working on Android Mobile with ES 3.1. Other features (like Raymarching) are tested: OK

(c) In NinjaLive 1.0 - 1.2 Tutorial levels contain Destruc  ble Meshes to demonstrate the

interac  on of fracturing meshes / fluid dynamics / LOD. During mobile packaging, UE returns a warning: "DM_box01_SphericalMapped has a LOD sec  on with 150 bones and the maximum supported number for feature level ES3_1 is 75." Since the referenced destruc  ble is not used on mobile levels, you could ignore the warning - it does not block compiling / packaging.

14.3 RANDOM HINT: we could imitate mobile touchscreen in Unreal Editor, by: Edit/Editor Preferences/Level Editor/Play/Play in Editor/Use mouse for touch: ON

15. Packaging To Android, Briefly

Useful official guide from EPIC: Se   ng up for development for the Android pla  orm

h  ps://docs.unrealengine.com/en-US/SharingAndReleasing/Mobile/Android/Ge   ngStarted/i ndex.html

15.1 Prerequisites

(a) On the PC: install CodeWorks for Android

(b) On the Mobile: Go to /Se   ngs /About device /So  ware Info /Build Number: tap 7   mes to enable developer mode. In the se   ngs root, search for "developer op  ons", enable USB debugging.

15.2 se   ng up the Unreal Editor for Android packaging

go /Edit /Project Se   ngs  /Packaging /Build Config: Developing OR Shipping go /Supported Pla  orms: set to Android

go /Edit /Project Se   ngs /Pla  orms / Android: Enable APK packaging

go /Se   ngs (cogwheel icon) /Preview Rendering Level: Android ES 3.1 (Shader (re)compiling!)

15.3 se   ng up Ninja for Android packaging

Android graphics drivers are flipping RenderTargets ver  cally. Since Ninja is using a chain of RenderTargets, only ODD pieces (seem to) flip! To prepare the system for this, go to: NinjaLiveComponent Details /LiveCompa  bility - and set "Flip RenderTargets for Mobile" to ON (before compiling).

15.4 Packaging

go /File /PackageProject /Android /...(ASTC)

Select a Folder for Android package and auto-installer (Subfolder is automa  cally generated)

(a) You could follow the packaging process in the Output window. (b) Once done, plug your phone, go to the above provided folder, and start "Install*.bat" - (c) A console pops up with status, and in a minute process the package is copied / installed on Android. (d) The package appears at the "All Apps" list with typic UE logo.

Se   ng up an Android Phone for developing / package transfer:

see UE documenta  on (enabling developer mode, allowing USB access at file transfer, finding the executable) + USB usage mode: charging VS USB for file transfer

16. Planned Features

UPDATE (21 Aug 2021): with NinjaLive version 1.5.26.1, the first round of adding core features has ended. Star  ng with 1 August 2021, we are focusing on producing prac  cal examples. The examples are developed in small packages: “UseCases” - see Chapter 29

LEGACY FEATURE PLANS: --- Short term

Enable memory management for SimplePainterMode blueprints

Status: blueprints in SimplePainterMode are currently autonomous (no memory manager connec  on)

Update the current Memory Manager to cope with mul  -resolu  on RenderTargets

Status: currently Mem.Manager generates and manages RenderTGs in a single, pre-defined resolu  on

Add PostProcess material output and support full 2D screen-space fluids Status: the current pipeline supports only geometry-mapped approach

--- Mid term

VR support

Further Niagara development Support for mul  ple light sources

Support for Foliage: distort geometry with simula  on velocity field Support for Virtual Textures

Adding more rendering-modes. Currently: (a) fluidsim, (b) simple painter. First add on candidate: (c) reac  on diffusion - see LINK

(d) simple feedback mode: the current simple painter extended with a 2 or 4 rendertarget feedback loop - no fluidsim func  onality, but able to deliver nice dynamic VFX

Implement "velocity based frame blending": calculate the sim on low FPS (5-25) and use the low frequency velocity data to smoothly interpolate between frames

Implement "capture unwrapped mesh" technique to improve collision painter Replace APEX destruc  on support with Chaos

--- Long term

Add 3D sim capabili  es

Add baking capabili  es / Merge NinjaLive and NinjaTools

17. Op  miza  on

17.1 Use Ac  va  on Volume

By using the "Ac  va  on Volume feature", you could reduce the number of parallelly running sim containers.

Performance data suggests that we could use 1-2 high res area fx and 3-6 character fx simultaneously (on one screen) without taking over too much resources reserved for other game components. For an in-game cinema  c the budget is more allowing: a single ultra HD (2k) - OR - 2-4 high res (1k) area fx - AND - 4-8 character effects.

Sim container sleep-wake state could be controlled at NinjaLive Actor Details panel /LiveAc  va  on.

When placing containers, please keep in mind: Live is not a robust "whole world system", but a system suppor  ng a mul  tude of dispersed, local interac  ons. See chapter 8 and 12 on performance.

17.2 Use LOD

Ninja is capable of distance-based quality reduc  on, regarding (a) sim sampling rate (FPS), (b) the number of pressure itera  on cycles and (c) the number of tracked objects. See: NinjaLiveComponent Details /LivePerformance

17.3 Use Memory Pool Manager

Place a single RenderTarget Pool Manager actor on a (persistent) level and allow clients (NinjaLiveComponent owners) to acquire memory from the pool. The op  on to enable this feature is located at:

NinjaLiveComponent Details /LiveMemoryManagement /AutoConnectToMemoryPool

Advantages: (a) Generate a pool of RenderTargets in advance, (b) Let waking / sleeping ninja components [acquire from] / [release to] pool, (c) Avoid lag caused by game  me rendertarget crea  on and destruc  on, (d) Spare unreal garbage-collector load

17.4 Reduce the number of Trace Targets

NinjaLive is line tracing overlapping objects to project their posi  on to the 2D sim plane.

The number of trace targets ma  er - with 10-20 objects, we are fine. As soon as the number of targets exceeds 50, we could experience a 10-15% drop in performance. Op  mize by reducing the number of tracked targets.

EXAMPLE1: DESTRUCTIBLES WITH MANY CHUNKS

Use the below op  on to make Ninja ignore a certain percentage (%) of chunks: NinjaLiveComponent Details /LivePerformance /IngnoreDestruc  bleMEshChunks%

EXAMPLE2: SKELETONS

Mannequin skeleton is made of 68 bones. Usually, we do not need them all - eg: for a full body collision we do not need all the fingers. By manually providing selected bone names at the below input field, we could cover the full body:

NinjaLiveComponent Details /LiveInterac  on /Con  nuousInterac  onBoneNamesExact

Tip: 14 Mannequin bones covering the full body (paste these list items to “BoneNamesExact” array)

head, neck_01, spine_01, spine_03, upperarm_twist_01_l, upperarm_twist_01_r, lowerarm_twist_01_l, lowerarm_twist_01_r, index_01_l, index_01_r, calf_l, calf_r, ball_l, ball_r

17.5 Disable Dynamic Asset Lookup (DAL)

Preset files could refer to bitmaps as density and velocity inputs. The preset stores only the bitmap's name and looks up the file dynamically, at sim ini  aliza  on. This gives flexibility to the system, we could easily swap images and experiment. On the other hand, we need to handle this, to run the game in Standalone mode and to Package the game:

(A) OPTIMIZATION

Once done with experimen  ng and a preset file is "final", the dynamic parsing/lookup feature is not needed anymore - we could bypass it and "hardwire" a bitmap input to the blueprint, using the below op  ons:

NinjaLiveComponent Details /LiveCompa  bility /Overwrite Preset Density Input and  /Overwrite Preset Velocity Input

The Issues and FAQ PDF contains a step-by-step guide on disabling DAL: see i005

(B) COMPATIBILITY

The velocity and density maps from the presets are dynamically loaded in real   me but the package system of UE4 doesn't consider dynamically loaded assets as assets it should cook in the final package - so it doesn't include it.

In case users do not want to use the "overwrite method" (op  on “A”), go to…

Project se   ngs /Packaging /Addi  onal Non-Asset Directories to Package ...and link the folder where the velocity and density maps were

(/Content/FluidNinjaLive/Presets), this forces the system to package the maps.

17.6 Carefully plan Rendertarget resolu  on

RenderTarget resolu  on influences performance. RT resolu  on could be scaled globally and selec  vely at:  NinjaLiveComponent Details /LivePerformance

17.6 Carefully choose the Pressure Solver (2 types) and solver se   ngs

Simula  on Pressure Field is calculated using an itera  ve process. More itera  ons result in more detail in turbulent structures / vor  city.  More itera  ons also need more RenderTarget read/write opera  ons per frame. Since RenderTarget write opera  ons have a cri  cal impact on performance, we are facing a TRADE OFF situa  on: performance VS structural details

Please have a look at this demo level: NinjaLive_Level02B_Cri  calSe   ngs - demonstra  ng the trade off. FIVE pressure itera  ons per frame (per render cycle) seems like a good compromise -

5 is set as default for NinjaLive. You could adjust this value at: NinjaLiveComponent Details /LivePerformance

17.8 Reduce the number of "per Tick" refreshed Dynamic Material Params

To increase flexibility during VFX development, by default almost all fluidsim params are queried / refreshed / forwarded to sim materials ON A PER TICK BASIS.

Once your project is about to finish / you are done with param tweaking, you could eliminate dynamic (per   ck) param refreshing by pu   ng non-changing values on a "do once" branch, or ini  alizing them on "EventBeginPlay".

Params like Noise   ling and offset are typically targeted for this op  miza  on: probably nobody is going to animate noise dynamically in game.

17.9 Blueprint Na  viza  on (will be deprecated in UE5!)

Turn on Blueprint Na  viza  on to improve the performance of COMPILED (packaged) project at Project Se   ngs /Packaging AND in the Ninja Core Blueprints, by opening the BP and visi  ng the Blueprint Class Se   ngs: Na  vize = TRUE

18. Changelog / Updates

This chapter is outsourced to a separate document: Link to PDF

19. Known Issues & FAQ

This chapter is outsourced to a separate document: Link to PDF

20. Niagara

Update, 2 June 2021 - Live 1.3 has been released with more Niagara Features. You could read about these features in the ChangeLog (LINK)

20.1 Driving par  cle systems using real   me fluid data

As the ini  al phase of planned Niagara feature development, a new Level (LV 20) has been added to NinjaLive at release v 1.0. The Niagara systems on this level serve as proof of concept: Niagara could be driven by externally generated, real   me fluidsim data.

All systems are using a 2D grid setup: par  cles are generated along an orthogonal grid and their ini  al posi  on is used as UV to map them with Fluidsim data, received from NinjaLive - by the Texture Sample Niagara Module.

Setup 1

Input: NinjaLive density and velocity buffers

Density is used to control par  cle Color and Alpha via "Scale Color" modules Velocity is used to control par  cle Accelera  on

Setup 2

Same as Setup1, except VERTICAL par  cle VELOCITY is modified by the Density buffer.

High density pushes par  cles upward. As a result, par  cles "burst" perpendicular to the 2D grid that spawns them and fall back as they "cool down".

Setup 3

Same as Setup1, but the NinjaLive preset that drives the simula  on is different: a persistent brush is used, so collision strokes remain in the sim area for a long   me, genera  ng long smoke trails.

Setup 4

Only NinjaLive Density data is used (no velocity) to drive Mesh Par  cle VERTICAL SCALE.

20.2 Warnings

(1) Niagara Plugin

Please note: under UE 4.23 / 4.24 the Niagara plugin should be ENABLED to view the systems on this level. In higher UE versions, the plugin is enabled by default.

(2) UE bug

Ninja-to-Niagara data flow is corrupted by a cri  cal UE 4.26.0 bug - as a temporary solu  on, we are using a workaround to send data to Niagara. You can read more about the UE bug at this LINK.

(3) Deprecated Niagara Systems

EPIC made many changes to Niagara since UE 4.23 came out. As we would like to maintain NinjaLive under UE

4.23 - 4.26 as a single, not-forked, compa  ble dev branch... we are using Niagara Systems made with UE 4.23. In case you are opening the project under UE 4.25 / 4.26, Niagara Editor is displaying warnings that some data types are deprecated. Despite the warnings, the systems are up and running - and the deprecated fields / modules could be swapped with up-to-date func  ons in a few minutes.

See video demonstra  ng Level 20: LINK

20.3 STEP-BY-STEP GUIDE: LINKING NIAGARA AND NINJA

NinjaLive is performing real-  me fluid simula  on. The sim buffers could be used to drive Niagara GPU-par  cle masses. For example: sim velocity field is ideal for accelera  ng par  cles, sim density could be used for opacity masking, height displacement and tone-mapping.

To do: we are wri  ng ninja internal simula  on buffer(s) to external RenderTargets and set a given NiagaraSystem to read these RenderTarget(s) as input

1.  Create one or more RenderTargets in advance

These are going to serve as a data-bridge to Niagara. Right click in the Content Browser, go to "Materials and Textures", choose "RenderTarget"

2.  Set ninja to write the RenderTarget(s)

At: NinjaLive Actor / NinjaLiveComponent /LiveGeneric

a.  switch on "Draw Internal RenderTargets to External"

b.  chose which buffers to export

c.  pick an exis  ng (previously created) empty RenderTarget to write

3.  Once the RenderTargets are created and ninja is set up to write them:

tell Niagara to read the RTs

a.  select a Niagara System on level that you would like to drive

b.  locate the actual uasset that is used (Details Panel /Niagara System asset)

c.  open the system with Niagara editor (double click on it in the Content Browser)

d.  locate the "Sample Texture" module in the Stack

e.  provide the pre-created RenderTarget as input in the "Texture" field

Ease of use: we could handle Niagara texture input as a parameter - so we can provide RenderTarget input on the Actor Details panel (no need to edit actual Niagara Systems when picking RenderTargets)

A.  create a Texture-Sample type "User Parameter" in the Niagara System

B.  provide this param as input to the "Sample Texture" module

C.  set the value of this param by selec  ng the on-level niagara system,

then going to the  / Details Panel / Overrides

21. Complete List of NinjaLive parameters

21.1 GUI exposed params & func  ons	47

21.2 NinjaLive Actor params	21

21.3 NinjaLive Actor Component params	94

21.1 GUI exposed params

Recalling chapter 6.4 on Preset Manager / Presets

By (1) placing /Content/FluidNinjaLive/NinjaLive_PresetManager on a level and

(2) pressing Play (“running the game”) you could acces a custom GUI side panel, layered over the level editor viewport, designed to manage

fluidsim related NinjaLive params:

A. Select the sim container that you would like to work with

B.  Modify param values real   me - while monitoring the changes in the viewport

C.  Load, save, duplicate and modify parameter collec  ons, stored as preset files

D. Set one the experimentally created preset files as default preset to a given NinjaLive instance on level - later on, the container is loading this default preset autonomously when ini  alized, no Preset Manager is needed

Please note: GUI exposed params are in fact NinjaLive Component params, that

have _not_ been exposed on the Blueprint Details panel - but made accessible for GUI

Second, blue roll down menu at the sidebar top PRESETS

NinjaLive is configured by 140 public variables

- Most variables (100) are exposed at blueprint details panel - Fluidsim specific params (40) are saved/loaded as a preset

2. Op  onally, templates could be referenced by absolute paths, (eg.: /Game/Textures/T_noise) by manually pas  ng the path to a DataTable (a preset file)

Check LEVEL 2A / STAGE 14 to see how it works.

Floppy disk icon

SAVE PRESET

Topmost, red roll down menu at sidebar top NINJALIVE SIMULATION CONTAINER ACTORS LIST

Quick browse list via keyboard up/down arrows. The selected Actor is highlighted on level.

Actors are listed IF matching the following criteria:

1.  placed on level

2.  enabled (non-disabled)

3.  containing NinjaLiveComponent

4.  equipped with NinjaLive Interface

Each container is associated with a default preset. This is marked with a star [*] besides the given preset.

Default preset could be changed (a) permanently at NinjaLiveComponent Details /LiveGeneric ... or

(b) temporarily by clicking the blue “star and arrow” picto on the right. This "temp default" condi  on is for preview only, to link a given preset to a given container while the user is changing back-and-forth between containers. The "temp default" se   ng is being erased as you quit NinjaLive GUI.

Presets are stored as DataTable uassets and could be modified (a) via Preset Manager or (b) by manually edi  ng the DataTables

Presets for a given instance of NinjaLive are collected from a predefined folder (and its subfolders). Preset lookup loca  on could be changed at

NinjaLiveComponent Details /LiveGeneric

Presets are recognized if… (a) Placed under pre-defined lookup loca  on (+ all subfolders) --- (b) Properly named: [FileType]_[ModuleName]_[PresetName]

Eg.: DT_NinjaLive_Flame1

Presets could reference other assets (density and velocity textures) and pull them as input for simula  on. Asset refs are called "Templates".

1. Normally, the preset contains only the name of the referenced asset and the preset loader looks for the template in the same folder where the given preset file is located - it is recommended to keep presets and their belongings together! (Preset Packages)

Instantly updates the original preset with current se   ngs. Note: before closing Unreal Editor,  updated preset files must be wri  en to disk permanently via "File / Save All"

Double floppy disk icon DUPLICATE PRESET	("Save as...")

Writes current se   ngs to a new file and asks for a file name. File naming: [FileType]_[ModuleName]_[PresetName]

Eg.: DT_NinjaLive_Flame1

Note: before closing Unreal Editor, newly created preset files must be wri  en to disk permanently via "File / Save All"

Floppy disk icon with line through REMOVE PRESET

Instantly removes preset from list. Default preset (with *) can't be removed

The removed asset is preserved as: DT_RemovedPreset_[original preset name]

Blue star with upward poin  ng arrow on top SET CURRENT PRESET AS TEMP DEFAULT

Sets the currently loaded preset as TEMPORARY default to the selected NinjaLive Component.

IMPORTANT: default could be set PERMANENTLY on the details panel of a given level actor. The "temp default" condi  on set via GUI control is for preview only - to link a given preset to a given container while the user is changing back-and-forth between containers. The "temp default"

se   ng is being erased as you quit NinjaLive GUI.

Op  onally, you could try pressing "K" on the keyboard - this calls the "Keep Simula  on Changes" UE func  on - tries to save the changes made  to an actor to the actor's default state. The UE func  on seems unstable.

Next we are going to describe GUI exposed numeric parameters.

NOTE ON PARAM OVERDRIVE

NinjaLive comes with parameter range-constraints that allow users to OVERDRIVE certain params. Reason: overdrive could look cool - in some cases. But it could also ruin the visuals.

See LEVEL 2B in the project - demonstra  ng overdrive.

Rule: params behave op  mal in the [-1]-[0]-[+1] range. GUI displays cri  cal params in RED (out of recommended range). Be careful when boos  ng them. This is especially true for velocity params, that behave addi  vely - and produce rude glitches if set to high values.

Parameters in the green “Brush” param group

Size

Blueprint variable: BrushSize Range: 0.01 - 5

Note: apart from the this GUI param, brush size could be selec  vely set for various object classes at: NinjaLiveComponent Details /LiveBrushSe   ngs

Eg.: a pawn draws 0.5 stroke, while primi  ves use 0.1 stroke

Density

Blueprint variable: BrushStrength Range: 0.001 - 10

Note1: a dense brush also draws stronger velocity signal Note2: when inver  ng the brush, density below 0.1

makes it prac  cally "invisible" while s  ll dragging the fluid.

Enhance this by se   ng "drag" above 3 Hardness

Blueprint variable: BrushHardness Range: 0 - 1

By increasing this, brush so   edge gets thinner

BrushNoise

Blueprint variable: BrushNoise Range: 0 - 1

Brush density noise

PosRnd

Blueprint variable: BrushRnd Range: 0 - 1

Brush posi  on noise

Persistence

Blueprint variable: InputFeedback Range: 0 - 1

Persistence sets brush stroke erosion, value = 1 means temporally stable, non-fading brush stroke.

Related param: simula  on "Feedback"

Invert

Blueprint variable: EraserMode

Bool switch, subtracts brush density from the sim density field. Low density makes brush "invisible" while dragging the fluid. To invert velocity, use nega  ve "drag" and "puncture"

Drag

Blueprint variable: VeloFromBrushMo  on Normal range: [-1] to [+1]

Overdrive range: [-20] to [+20]

OVERDRIVE MIGHT CAUSE GLITCHES, BE CAREFUL! Adds the brush-masked velocity of colliding objects to the simula  on. Nega  ve values invert direc  on.

Puncture

Blueprint variable: BrushPuncture Overdrive range: [-2] to [+2] Recommended range: [-0.1] to [+0.1]

Puncture adds radial velocity to the brush, modulated with strong noise. The result is a "boiling" effect - designed for sta  c or slow moving objects, that otherwise would not produce visually interes  ng turbulent advec  on.

Parameters in the yellow “Sim” param group

Feedback

Blueprint variable: FlowFeedback Range: 0 - 1

Live render pipeline is a feedback loop. Feedback = 1 means no dampening, no fade-out, density informa  on accumulates.

Related param: brush "Persistence"

Clear buffers

Clear all RenderTargets in the pipeline / blank Canvas

Speed

Blueprint variable: Speed Range: 0.01 - 10

Simula  on state is stored in RenderTargets. The spa  al propaga  on of density/velocity informa  on is the result of sampling RenderTargets with a given offset. "Speed" is a sampling-offset mul  plier: larger values result in greater offset and faster propaga  on. Note: sampling frequency

(  mestep) also influences sim speed.

Turbulence

Blueprint variable: Divergence Range: 0.01 - 1.3

Velocity field divergence modifier

Larger values result more turbulent flow

Parameters in the purple “Fields” param group

Amplitude

Blueprint variable: VeloStrength Range: -/+

Velocity field strength mul  plier Nega  ve values invert direc  on

Has _no_ effect on "drag" and "puncture"* *(stroke velo separated from field velo)

Offset X

Blueprint variable: VeloOffsetX Range: -/+

Adds horizontal force to velo field

Offset Y

Blueprint variable: VeloOffsetY Range: -/+

Adds ver  cal force to velo field

Amp.Noise

Blueprint variable: VeloAmpNoise Range: 0 - 1.5

Adds noise to velocity field amplitude _No_ influence on velo direc  on Intended use: introduce inhomogeneity, dampen strong direc  onal forces

Dir.Noise

Blueprint variable: VeloDirNoise Range: -/+

Adds curl noise to velocity field direc  on Intended use: introduce swirling

DirNoiseSize

Blueprint variable: VeloDirNoiseSize Range: 0.01 - 10

Curl noise scale

DirNoiseOffs

Blueprint variable: VeloDirNoiseSpeed Range: -/+

Curl noise dynamics offset

Any non-zero value animates the offset

Velocity Field from Bitmap

Texture based velocity field template, that could be transformed by "Rotate" / "Tile" and animated by "Offset"

Note1: By default, the preset loader looks for the template in the same folder where the given preset file is located.

Op  onally, templates could be referenced by absolute paths (eg.: /Game/Textures/T_noise) by manually pas  ng the

path to the preset DataTable file in Content Browser. Check LEVEL 2 / STAGE 14 to see how it works.

Note2: template references could be "hardwired", transferred from the preset file directly to the blueprint. See Manual (Help) 17.5: DISABLE DYNAMIC ASSET LOOKUP

Warning, yellow triangle

Occasionally you could see a yellow triangle besides the Texture input field.

This is a warning: the referenced bitmap is NOT in the same folder as the preset file / is referenced with a manually inserted absolute path.

The path is not auto-upda  ng: in case the referenced texture asset is moved/deleted the link is broken.

Rotate

Blueprint variable: VeloRotate Range: 0 - 1

Rotate velocity field vectors

0.25 means 90 degree, 1 is a full turn

Offset

Blueprint variable: VeloInputOffsetSpeed

Range: [-1] to [+1]

Velo field Dynamic offset

Any non-zero value animates the offset

Tile

Blueprint variable: VeloInputTile Range: -/+

Velo field   ling: 0 = default, x > 0 means zooming out

x < 0 means zooming in

Offset From Sim Area Mo  on

Blueprint variable: OffsetFromSimAreaMo  on Range: -/+

Global iner  al frame of reference: sim container WorldSpace posi  on delta is used to offset simula  on buffers.

Useful when moving objects leave trails or marks behind.

Velocity Field influenced by Sim area mo  on Blueprint variable: VeloFromSimAreaMo  on Range: 0 - 1

Global iner  al frame of reference: sim container WorldSpace velocity is subtracted from the simula  on velocity field.

Fluid behaves inertly when the area moves to a certain direc  on. The iner  a effect could be further enhanced by: NinjaLiveComponent Details /LiveInterac  on /LegacySimAreaMo  onEffectsSimDensity

Tile

Blueprint variable: DensityTxtScale, Range: -/+

Density field   ling: 0 = default, x > 0 means zooming out, x < 0 means zooming in

Density Field from Bitmap

Texture based density field template, that could be transformed by "Scale" and animated by "Offset" (below)

Note1: By default, the preset loader looks for the template in the same folder where the given preset file is located.

Op  onally, templates could be referenced by absolute paths (eg.: /Game/Textures/T_noise) by manually pas  ng the

path to the preset DataTable file in Content Browser. Check LEVEL 2 / STAGE 14 to see how it works.

Note2: template references could be "hardwired", transferred from the preset file directly to the blueprint. See 17.5: DISABLE DYNAMIC ASSET LOOKUP

Offset X

Blueprint variable: DensityTxtOffsetX, Range: -/+ Density field Dynamic horizontal offset

Any non-zero value animates the offset

Offset Y

Blueprint variable: DensityTxtOffsetY, Range: -/+ Density field Dynamic ver  cal offset

Any non-zero value animates the offset

Density Field from Material   [Preview]

Simula  on density could be driven by (animated) Materials

Important: these materials are here to PREVIEW / DEBUG, and the actual selec  on is _NOT_ stored in the preset. Input Materials list and defaults could be managed at: NinjaLiveComponent Details /LiveGeneric

See LEVEL3 /STAGE 4-5-6 for examples

See /Content/FluidNinjaLive/InputMaterials folder for example materials

Density Field Params

Amplitude

Blueprint variable: DensityTxtMult Range: 0 - 10

Scales density field amplitude, both texture and Material input

Amp.Noise

Blueprint variable: DensityInputNoiseAmp Range: -/+

Adds a cloud-like mul  scale noise to the density field

NoiseOffset

Blueprint variable: DensityInputNoiseOffset Range: -/+

Density field Dynamic offset

Any non-zero value animates the offset

NoiseTile

Blueprint variable: DensityInputNoiseTile Range: -/+

Density field noise   ling: 0 = default, x > 0 means zooming out

x < 0 means zooming in

Sim area border is colliding with fluid Blueprint variable: SimEdgeBouncyness Range: 0 - 1

Makes sim area border behave like a wall, stopping / reflec  ng fluid dri

Sim area border density fading (Pow) Blueprint variable: FadeDensityAtSimEdge Range: 0.01 - 1

Kills density near the sim edges

Sim area border density fading (Width) Blueprint variable: EdgeMaskWidth Range: 0.05 - 1

Defines the width of sim edge area, where density is being killed --- 0.5 means "from edge to center"

Output:  filter sim output via Material [Preview] User defined Output Material, based on arbitrary combina  on of simula  on buffers.

Important: these materials are here to PREVIEW / DEBUG, and the actual selec  on is _NOT_ stored in the preset. Output Materials list and defaults could be managed at: NinjaLiveComponent Details /LiveGeneric

Tip1: have a look at a base material to see how it works, eg.: /Content/FluidNinjaLive/OutputMaterials/BaseMaterials/M_ NinjaOutput_Basic

Tip2: OutMaterial Instances could be edited real   me, while NinjaLive / Gameplay Scene is running, a quick way to fine-tune colors, alpha...etc.

SavePaintBuffer

Event Dispatcher: CallOnDensityMapSave Save density and velocity paint buffer

to the same folder where the preset is located

SaveSimBuffer

Event Dispatcher: CallOnDensityMapSave Save density and velocity simula  on buffer

to the same folder where the preset is located

MetaInfo

Blueprint variable: MetaData

Add your notes, instruc  ons, author data here

END OF GUI PARAM DESCRIPTION NEXT: NINJALIVE ACTOR PARAMS

21.2 NinjaLive Actor params

When selec  ng a "NinjaLive" Actor on level, the Details panel is displaying exposed variables, collected in FOUR groups

LiveAc  va  on: [9 params]

disable switch + wake/sleep related params LiveInterac  on: [12 params]

scale sim area + filter interac  on types LiveDebug: enable on screen status messages

LiveComponentOverrides: helper func  on to set variables when mul  ple actors are selected.

21.2.1  LiveAc  va  on

DisableBlueprint

Disabled sim containers do not ini  alize.

SimAc  vatedByPawnProximity

Container "disabled" state is controlled by the proximity of a user defined, arbitrary object (called "Ac  vator"). If NO

Ac  vator is defined: using actually possessed pawn.

If container is disabled while running (eg. the Ac  vator le the ac  va  on area), state is not preserved, container shuts down, memory returned to the manager. Re-ini  alized by Ac  vator proximity (by calling "RePlay").

ShowAc  va  onVolumeInEditor Show Ac  va  on Bounds

Ac  va  onVolumeSize Set Ac  va  on Bounds

Ac  vatorProximityCheckFrequency Ac  va  on response   me:

the frequency of Ac  vator proximity tracking.

Default value is 0.1 second.

Ac  vatorType

Ac  vator Collision Object Type (eg. Pawn, Vehicle, Dynamic)

Ac  vator

User defined agent, ac  vates given container by proximity

TraceMeshInac  veBehaviour Visual proper  es of Disabled state

21.2.2  LiveInterac  on

ShowTraceMeshInEditor

(1) Fluid sim Output Material is mapped on TraceMesh

(2) Line trace is performed against TraceMesh to project 3D collision data to the 2D sim

TraceMeshSize

Default TraceMesh is a planar mesh. For this reason, when you set size, the Z component of the input field does not have any effect on the visuals and workings.

You can replace TraceMesh with a non-planar object, and the Z-scaling factor could be used.

UserInputBasedInterac  on

If disabled, mouse and touch gestures are ignored

OverlapBasedInterac  on

If disabled, collision/overlapping is ignored

UseTraceMeshAsInterac  onVolume (added to Live 1.1!) Choose between two objects for overlap detec  on: Interac  onVolume (default) and TraceMesh

Interac  onVolume is prac  cal when we would like to track moving objects in 3D. TraceMesh is advised when we need precise surface-hit posi  ons. Note: TraceMesh "Collision Complexity" also ma  ers - see FAQ doc for details.

ShowInterac  onVolumeInEditor Show/hide volume

Interac  onVolumeSize

Interac  onVolume detects collision/overlapping. By default, its X,Y size should be the same as the TraceMesh. It is a true 3D volume, Z extension also influences behaviour. Z-scale enables the sim container to detect objects in front of and behind the TraceMesh 2D plane.

OverlapFilterInclusiveObjType

Inclusive filter for Collision Object Type: only listed classes could cause overlap events

AutoExcludeLargeOverlappingObjects

Ignores colliders that are larger than the sim area

ExcludeSpecificActorsFromOverlap

Name specific actors to exclude them from overlap detec  on

OverlapFilterInclusiveBoneNamePar  al

Type strings like "feet" or "hand" to include all bones containing the given string in their name

OverlapFilterInclusiveBoneNameExact

Name specific bones to inclusively track them by the overlap detec  on mechanism.

This is the suggested way to handle skeletal mesh collision.

IgnoreDestruc  blesBeforeFracture

Destruc  bles need a hit event to start falling apart. NinjaLive could ignore destruc  ble chunks,

before they start to fall apart.

21.2.3  LiveDebug

Ac  va  onEventsDebugPrint SaveDebugTextToLog DebugTextLifeTimeLength

21.2.4  LiveComponentOverrides

(14 params)

21.3 NinjaLive Actor Component params

Recalling Chapter 10.2

Select any NinjaLive-Component-Owner Actor on level Select NinjaLiveComponent at the Actor Details panel. Here is a visual guide on selec  ng NinjaLiveComponent: IMAGE LINK

No  ce: as you select the actor component, the "Details" panel changes, revealing Component Variables, collected in 9 groups -  listed in order of importance:

LiveInterac  on [13 params] - con  nuous (non-overlap based) interac  on, single target mode LiveGeneric [14 params] - preset and input / output material defini  ons

LivePerformance [19 params] - resolu  on, LOD, render pipeline se   ngs LiveMemoryManagement [3 params] - set up memory pool manager connec  on LiveRaymarching [16 params] - picking lightsource for raymarching, se   ng params LiveCompa  bility [6 params] - system level switches, eg.: flip RenderTargets for mobile LiveBrushSe   ngs [11 params] - collision painter brush size overrides and noise se   ngs LiveAc  va  on [1 param] - component level disable switch

Debug [10 params] - enable on-screen repor  ng of LOD, memory...etc

21.3.1  LiveInterac  on

Con  nuousInterac  onWithOwnerActor THIS FLAG IS OBLIGATORY ON, IN CASE

NINJALIVECOMPONENT IS EMBEDDED TO A CUSTOM OWNER. (custom = NOT NinjaLive Actor) NinjaLiveComponent does _not_ have built in overlap

detec  on func  onality, interacts only with predefined (user defined) owner components (eg. Bones, Sockets, Meshes). This type of interac  on is labeled "non-overlap based" or "con  nuous" interac  on, handled by MODULE021 in the NinjaLiveComponent blueprint.

Con  nuousInterac  onInclusiveObjType Inclusive filter for Collision Object Type

Con  nuousInterac  onBoneNamesExact Inclusive filter for specific bones/sockets

SingleTargetMode

STM is a special mode of Collision Painter, op  mized to draw the mo  on trajectory of very fast objects, by interpola  ng between available posi  on samples.

See LEVEL 2 / STAGE 3.

LIMITATION: only a single object / bone could be tracked

SingleTargetType

Primi  ve OR Bone/Socket. Rule: the target is the first item in the   "Con  nuousInterac  onBoneNamesExact" list or the first primi  ve element found by type filtering.

SingleTargetMoveSetSimSpeed

A bullet-  me like "  me dila  on effect" could be achieved: the moving speed of the single target influences fluidsim play speed

See LEVEL 2 / STAGE 5

SpeedInfluenceFactor

Blend between normal sim speed and "  me dila  on effect" defined speed.

0 = 100% normal speed, 1 = 100% dilated speed

CameraFacingTraceMesh

Forces the TraceMesh to con  nuously look towards the Player Camera. TraceMesh transform is performed by a shared, public func  on, stored at: /Content/FluidNinjaLive/Core/NinjaLiveFunc  ons

Important: when using NinjaLive as a component IN MOVING AGENTS (eg.: a Pawn), keep Camera Facing in the sim component switched OFF and call "Camera Facing" func  on in the owner Blueprint's Tick flow. This results in smoother rota  on. Reason: the   cking of Pawn transform and the TraceMesh transform should be synchronized.

CameraFacing_LockY-Axis

TraceMesh Ver  cal axis locked (forced to remain ver  cal). Result: a "billboard" like camera facing.

TraceMeshTranslucentSortPrio

A. In case you’d like to priori  ze a translucent tracemesh against other translucent objects:

NinjaLiveComponent /LiveInterac  on /TraceMeshTranslucentSortPrio - set prio here.

B. In case you’d like to force translucent TraceMesh in front of everything, including opaque objects: you need to modify an Output-Base-Material property. Go to /Content/FluidNinjaLive/OutputMaterials/BaseMaterials , pick the related basemat, enter mat.editor, in the mat. proper  es panel (on the right) find: "Disable Depth Test". Note: in the Material Instance (using the given base.mat), the "Depth Fade Switch"

must be OFF!

UseCustomTraceSource	(added to NinjaLive 1.2)

Force NinjaLive to use a USER DEFINED Line Trace source POSITION - instead Of Camera posi  on.

Explained in Chapter 10.2.6 + see Level 2C in the project.

CustomTraceSourcePosi  on	(added to NinjaLive 1.2)

The custom line trace posi  on is RELATIVE to NinjaLiveComponent OWNER posi  on. Add offset here.

LegacySimAreaMo  onEffectsSimDensity

This feature is meant to support "Velocity Field influenced by Sim area mo  on" FX (available on the Preset Manager GUI) - see GUI param descrip  ons for more info!

21.3.2  LiveGeneric

DefaultPreset

This is where you could associate a default preset for a NinjaLiveComponent. This preset will be automa  cally loaded at sim area ini  aliza  on. Note: in Preset Manager, you could easily load / swap presets as you develop visuals, but you have to provide DEFAULT here, at the Component Details.

PresetSearchPath

Presets for a given instance of NinjaLiveComponent are collected from a predefined folder (and its subfolders). Preset lookup loca  on could be changed here. This could be used to separate presets.

PresetNameFilterCriteria

Presets in the lookup folder are recognized IF properly named.

The name consists of THREE parts: [FileType]_[ModuleName]_[PresetName]

Part 1 is fixed (DT, DataTable).

Part 2 could be changed here - this tag could be used to make presets INCLUSIVE for a given container - so they will be displayed in Preset  Manager ONLY when this specific container is selected / they won't show up for other containers.

Part 3 is arbitrary, displayed as a preset name on the PresetManager GUI.

Sugges  on: this method also could be used to separate hundreds of presets to groups.

OutputMaterialInvisible

Forces Null Material on TraceMesh: all func  ons work (including collision) except the TraceMesh is not visible. Used when driving Volumetric containers with ninja - hiding 2D sim - so only the volume is visible

OutputMaterialSelected

The index of DEFAULT Material in the "OutputMaterials" array (see below)

OutputMaterials

An "Output Material" is (1) the final stage of NinjaLive Rendering pipeline, (2) mapped on the simula  on TraceMesh, (3) used to COMPOSITE simula  on buffers to a final visual, set colors, translucency, shading and such.

NinjaLive rendering pipeline is a chain of RenderTargets, all accessible for "Output Materials". Since Output Materials are user editable, many versions could co-exist --- this array func  ons as a "my favourites" selec  on: users could add elements to the list --- and all these elements are listed on the Preset Manager GUI, where they could be easily swapped, compared, used to check RenderBuffers...etc.

Pick output material examples from /Content/FluidninjaLive/OutputMaterials

InputMaterials

Simula  on density could be driven by (animated/dynamic) Materials. Since Input Materials are user editable, many versions could co-exist, this array func  ons as a "my favourites" selec  on: users could add elements to the list --- and all these elements are listed on the

Preset Manager GUI, where they could be easily swapped for tes  ng.

See LEVEL3 /STAGE 4-5-6 for examples

See /Content/FluidNinjaLive/InputMaterials folder for example materials

InputMaterialSelected

The index of DEFAULT Material in the "InputMaterials" array

RGBInputMaterial

Simula  on could be driven by user defined materials. Default: using material RED channel to drive simula  on density --- IF the RGB flag is TRUE, Ninja uses Material BLUE channel for density, and RG for velocity

InputSceneCaptureCamera

Simula  on density could be driven by SceneCapture Camera. IF a valid camera is provided, its input OVERRIDES Material based input. Visit LEVEL 3 / Stage 7 to see how it works!

InputMediaPlayer

Simula  on density could be driven by Local or Streaming Video, you could link your pre-configured media player here.

InputMediaSource

Simula  on density could be driven by Local or Streaming Video, you could link your media file pointer here.

SimAreaClamp

Simula  on RenderTarget handling method: Clamp or Wrap

RandomizeNoiseOffsets

In case you have mul  ple containers placed nearby and would like to avoid visual similarity, you could randomize noises.

RandomizeDensityTextureOffset

In case you have mul  ple containers placed nearby and would like to avoid visual similarity, you could randomize density texture offset. Pre-requisite:   leable density texture

SimplePainterMode

NinjaLive has the poten  al to track objects, detect collisions and write the data to a RenderTarget - this is what we call "Collision Painter".

SimplePainterMode (a) discouples Collision Painter from the rest of the system and (b) disables NinjaLive fluidsim

func  ons.

The result is a minimal system using only 1 RenderTarget, keeping GPU load on a very low level. Combined with the regular "User Editable Output Materials" feat, SimplePainter could be used efficiently for certain VFX types.

See 6.4 in the Manual / Help.uasset See LEVEL 7

DrawInternalRenderTargetToExternal

Draws a selected simula  on buffer to a user defined external, on disk RenderTarget uasset, per   ck.

New feature (introduced in Live 1.5): not only simula  on buffers, but the user defined OutputMaterial could be wri  en to external RenderTarget as well.

InternalRenderTargetsToExport

Source buffer for drawing / saving

ExternalRenderTargets

Target file for drawing / saving

CollisionMask

Use textures to define sim area shape. Collision Mask is a Black-and-White texture with "Alpha'' type encoding. Black areas repel fluid-currents and behave like an obstacle. Works ONLY if "Sim-area-border-is-colliding-with-fluid" Preset Parameter is set to 1. Ideal for masking sta  c environmental objects - eg: an altair in the middle of a fog-covered area.

AllowAbsoluteBlackDensity

Absolute ZERO black density input is causing visual artefacts when used as input for Parallax Occlusion Mapping. For this reason, Ninja uses "very dark gray" as density bo  om.

By flagging this op  on, 0-black could be forced.

PressureEdgeMasking

Range 0-1 --- When n>0, forces the pressure-buffer values to zero near the sim edges. Useful when pressure projected on a larger surface - eg. when making local ripples on a global water-surface

CoreSimMaterials

This array references ALL core simula  on materials. You

could op  onally replace them, in case you are experimen  ng with your own core material versions.

21.3.3  LivePerformance

Resolu  onX

Sim horizontal resolu  on

Important: in case the container acquires RenderTargets from the MemoryManager, the local resolu  on se   ngs are OVERRIDDEN by mem.manager resolu  on se   ngs!

Resolu  onY

Sim ver  cal resolu  on

UsePressureSolver1 (Default is 2)

Wri  ng RenderTargets is a performance bo  leneck in UE --- Pressure Solver1 is performing calcula  ons using many RenderTarget writes --- Pressure Solver2 is op  mized to keep the number of RT-writes low ------ Un  l the sim.resolu  on is below 2K pixel, Solver 2 is superior. Above 2K, Solver 1 starts to perform be  er.

PressureSolver1_MaxItera  ons

Range: 1-32 --- Default is 5 --- Pressure Solver 1 visual quality could be improved by performing more itera  ons (more RenderTarget writes)

PressureSolver2_MaxItera  ons

Range: 1-4 --- Default is 1 --- Pressure Solver 2 is op  mized to keep RT-writes low. More itera  ons result slightly more details - but (1) SUBOPTIMAL and (2) sensi  ve to Divergence Overdriving --- use only for CINEMATIC rendering --- See LEVEL 2B for visual tests

PressureSolver2_KernelReduc  on

Pressure Solver 2 checks large neighborhood areas around a given texel to calculate average values. By reducing "Kernel size", the number of examined neighbours could be crunched --- increasing performance, but and altering visual quality

MaxSamplingFPS

The maximum number of fluidsim cycles per second. LOD scales fluidsim between min/max sample ranges. The value also constraints "CollisionPainter" temporal sampling: lower sampling causes fast moving objects

pain  ng "discrete points" instead of con  nuous lines - this could be avoided by switching on "Single Target Mode" (STM) at NinjaLiveComponent Details /Live Interac  on. STM is drawing lines between sparse sample-points

MinSamplingFPS

The minimum number of fluidsim cycles per second. LOD scales fluidsim between min/max sample ranges.

HalfResPressureAndDivergenceBuffers

Calcula  ng pressure on lower resolu  on is op  mal, but the method is sensi  ve to simula  on SPEED adjustment: in case we increase sim speed above 1 (while HalfRes is ON) visual glitches emerge --- Note: in case a given container is receiving sim buffers (RenderTargets) from the Memory Pool, the Memory Manager RESOLUTION se   ngs overwrite local values (including the HalfRes flag)

LOD1-ReduceItera  ons

Allow view-distance based reduc  on of fluidsim pressure-subcycles

LOD2-ReduceSamplingFPS

Allow view-distance based reduc  on of fluidsim cycles per second. Min-Max ranges are defined by "Min/Max SamplingFPS" params

LOD-FarBound

Set LOD max distance: outside this area the LOWEST sampling values are used. Note: this se   ng is NOT related to ac  va  on volume size

LOD-NearBound

Set LOD min distance: within this area the HIGHEST sampling values are used

Note: this se   ng is NOT related to ac  va  on volume size

LOD-CheckFrequency

LOD response   me (frequency of view-distance change tracking) - default value is 0.1 second.

PauseSimWhenNotVisible

Pauses (freezes) sim when the player is not looking at it, and con  nues when focus returns. State is preserved while paused (as opposed to "Disable", that erases state)

WaitBeforePause

Time before pausing the sim (when out of player focus)

SimSpeedAdjustmentLatency

As LOD reduces sampling FPS, the fluid would seemingly slow down.

To counter this, we are increasing simula  on RenderTG SampleOffset, so density and velocity informa  on propagates faster.  At the end, sim remains normal as LOD kicks in - Default value is 0.1 sec

Important: when "Bullet Time FX" is on ("SingleTargetMoveSetSimSpeed"),

Sim speed adjustment is controlled by collider velocity!

StopUsingPainterCanvasWhenIdle

Halts collision painter RenderTarget overdraw when NO collision

Experimental_PSolver2KernelIndexOffset

NinjaLive Pressure Solver v2 uses TWO type of kernels: (A) Gaussian for the feedback blurring of Pressure Buffer and (B) Custom for the blurring of primary, raw Divergence data --- Default Kernel size is 15 --- KernelIndexOffset adjusts Gauss Kernel size, rela  ve to the Custom Kernel size --- Eg: a "KernelReduc  on" mul  plier of "0.5" reduces both Kernel to

7 (from 15) - and a "KernelIndexOffset" of "-2" sets the Gauss Kernel to "two level higher" to size "11" --- this param is experimental, allows us to seriously ABUSE the pressure solver - use carefully!

Experimental_PressureFeedback

There is a feedback component in the pressure calcula  on. "PressureFeedback" is a mul  plier in this process, lower values dras  cally kill the feedback component, resul  ng short lived pressure waves

21.3.4  LiveMemoryManagement

AutoConnectToMemoryPool-IF-Found

Search for Memory Manager and connect when found

SelfService-IF-PoolEmpty

ON: container could manage its memory autonomously OFF: container relies on Memory Manager and skips

ini  aliza  on if pool is empty or no manager found

21.3.5  LiveRaymarching

EnableRayMarching

To set up NinjaLiveComponent for Raymarching: (1) enable this switch, (2) please make sure that you have a "Raymarching capable" material added to this list: LiveGeneric /OutputMaterials and (3) the material set as container DEFAULT via "OutputMaterialSelected" index

LightDirec  onProvider

Pick any object (not necessarily a light source): its transform (posi  on OR rota  on) will be used to calculate raymarched self-shadows direc  on.

LightDirec  onSourceIsRota  on_NOT_Pos

If TRUE, LightDirec  onProvider's ROTATION is used to determine light direc  on. If FALSE, LD.Provider's rela  ve POSITION is used as light direc  on

DistanceBasedLightA  enua  on

If TRUE LightDirec  onProvider's posi  on is used as center, to map light strength with a radial gradient

A  enua  onPower

Power func  on applied on light a  enua  on gradient

TwoSidedShading

If TRUE, the light direc  on angle is weighted by TraceMesh up vector

TwoSideBlendPow

Influences the transi  on between "light" and "dark" sides of TraceMesh

OffsetLightVector

Offset the direc  on of light vector with a constant

ForceManualSunPosi  on

If TRUE, user provided light-source direc  on is used

SunLa  tude

User defined light source direc  on, X component

SunLongitude

User defined light source direc  on, Y component

SunHeight

User defined light source direc  on, Z component

21.3.6  LiveCompa  bility

Note: these params are extremely important, properly se   ng them guarantees that compila  on and cooking produces correctly working executables / apk files!

PreferredTraceChannelName

User defined Line Trace Channel NAME for a given container. The container automa  cally tries to find the defined channel using the Project Se   ngs. In case AUTOFIND fails, the container uses the values set in the TraceChannel and CollisionChannel ENUMS (below) as a "fallback" op  on. Default value = “FluidTrace”

Experience shows that the na  ve UE func  on used to search / look up trace channels works fine in PIE and Standalone mode, and DYSFUNCTIONAL in compiled exe/apk files. For this reason, it is very important to set the two “fallback” variables before project compiling.

TraceChannel and CollisionChannel

These variables store a trace channel index: “1”

The index is NOT shown: the UI representa  on displays the project defined Trace Channel NAME belonging to index 1. Once Ninja is merged to another project, it is important to (re)set this variable to an index that is associated with the “FluidTrace” Trace Channel.

To put it simple: set these variables to “FluidTrace”

SimPrecision

Fluid sim data precision bit depth: 8/10/16/32.

8/10 bit modes are experimental and need quite different sim params to produce nice results.

FlipRenderTargetsForMobile

Android graphics drivers are flipping RenderTargets

ver  cally. To counter this, developers need to "pre-flip" RenderTGs BEFORE packing the project to mobile / VR

UseUnrealNa  veEventTick

NinjaLive (by default) generates its own   ck signal, but could be forced to use Unreal Engine   cking. See NinjaLiveComponent Blueprint / BLOCK2 / MODULE027, 028

LimitUnrealNa  veEventTick

Set max value for   ck frequency (  ck per second). This affects NinjaLive when "UseUnrealNa  veEventTick" is on.

OverwritePresetDensityInput and OverwritePresetVelocityInput

Important: Live uses dynamic asset lookup in PIE to access simula  on input TEXTURES (density and velocity maps). Dynamic asset lookup DOES NOT WORK in STANDALONE mode and in COMPILED EXECUTABLES / APKs.

(Re-phrasing: the velocity and density maps from the presets are dynamically loaded in real   me but the package system of UE4 doesn't consider dynamically loaded assets as assets it should cook in the final package - so it doesn't include it.)

To provide a given container with the needed input textures in Standalone mode / following compila  on, there are two methods:

METHOD1: we need to properly fill the above  input fields. METHOD2: we need to force UE to cook the texture folder

Read more about texture inputs, dynamic asset lookup and Standalone mode --->

In this document:

14. Packaging / Compiling

17.5 Disable Dynamic Asset Lookup (DAL)

In the Issues and FAQ PDF: see i005 (issue 005)

21.3.7  LiveBrushSe   ngs

BrushScaledInverselyByTraceMeshSize

Brush size could be inversely scaled by canvas (TraceMesh) size. Result: a bigger canvas results in a similar brush as a small one.

BrushScaledByInterac  ngObjSize

Brush size could be scaled by colliders: smaller object draws thinner stroke

GlobalBrushScale

Scales all other se   ng

UserInputBrushScale

Brush size modifier for user gestures

Primi  veObjBrushScale

Brush size modifier for Primi  ves

SkeletalMeshBrushScale

Brush size modifier for SkeletalMeshes

Destruc  veBrushScale

Brush size modifier for Destruc  bles

BrushDensityNoiseScale

Sets CollisionPainter BRUSH default density noise scale

BrushDensityNoiseFreq

Sets CollisionPainter BRUSH default density noise anim-freq

BrushVelocityNoiseScale

Sets CollisionPainter BRUSH default velocity noise scale

BrushVelocityNoiseFreq

Sets CollisionPainter BRUSH default velocity noise anim-freq

DampenBrushBelowThisVelocity

Range: 0-1 --- By default, Brush is "Always ON" un  l a target overlaps --- Using this THRESHOLD VALUE, we can FADE or KILL brush below a given velocity --- Example1: a pawn walks in a fluid, and we do NOT want to generate waves or turbulence when the pawn stands - set value to 0.01 --- Example2: we would like to scale/fade the brush with velocity con  nuously - set value to 1 ------- ZERO means the brush is always on, and not scaled by velocity

DampenBrushFactor

Range: 0-9 --- This value influences how velocity scales brush density --- Near-zero values mean: even the smallest velocity causes MAX brush density --- Large values mean: only the highest velocity values scale up brush density

BrushVelocityPow

Range: 0.1 - 9 --- Control how raw brush velocity is being forwarded to the sim --- n=1 means no change, n>1 mean lower velocity values get killed, n<1 means lower velocity values get scaled up

SimAreaMo  onEffectsBrushPuncture

The Z component of sim area mo  on (perpendicular to TraceMesh) mul  plies PUNCTURE

21.3.8  LiveAc  va  on

DisableComponent

If NinjaLiveComponent is embedded in NinjaLive actor, this switch is overridden by Actor level disable. In case you embed LiveComponent to your custom class, use this to disable the system.

21.3.9  LiveDebug

ShowDebugMessages-MemoryManagement

Enable on-screen debug messages for mem.management

ShowDebugMessages-CollisionAndTracing

Enable on-screen debug messages for collision and line tracing

ShowDebugMessages-LODIni  al

Enable ini  al on-screen debug messages for LOD

ShowDebugMessages-LODRun  me

Enable con  nuous on-screen debug messages for LOD

ShowDebugMessages-InterfaceControl

Enable on-screen debug messages for NinjaLive Interface events

ShowDebugMessages-RenderTargetExport

Enable on-screen debug messages for render buffer expor  ng

SaveDebugMessagesToDefaultLog

Saves all debug messages to UE default log text file

DebugMessagesLife  me

Define life  me of on screen debug msgs

ShowMouseCursor

This se   ng is overridden by NinjaLive U  ls, Preset Manager and other containers.

(effects cursor only when a single container is placed on level, without managers)

ShowVelocityDebugCone

Displays a velocity direc  on oriented debug wireframe arrow of the given collider

IgnoreInterfaceCommands

NinjaLive_InterfaceController commands could be ignored.

Chart 2  Fluid simula  on: data flow. High.res bitmap: LINK

22. Live fluidsim pipeline: technical descrip  on

Update on 2 June 2021  -   NinjaLive 1.3 released

To improve PC and Mobile performance, a new pressure solver has been introduced and simula  on pipeline data flow has been modified.

22.1   About the new Pressure solver	(briefly)	22.2   About the data flow changes, briefly

Detailed descrip  on in the ChangeLog PDF, Live 1.3: LINK

Wri  ng RenderTargets is a performance bo  leneck in UE. Pressure Solver v2 changes the balance in a tradeoff situa  on - Compared to v1 solver -  v2 performs more

opera  ons per pixel and less RenderTarget read-write ops (see Level 02-B for compare)

All Pressure-solver related params could be accessed at:

NinjaLiveComponent Details Panel /LivePerformance (see Tool  ps for details!)

Comparing the performance of v1 / v2 pressure solvers  (Live 1.2 vs 1.3) GTX 1070, UE 4.23, average FPS

Level 11: SmokeChamber test scene   FOUR 720px fluidsim containers

A. Density and Velocity pipelines have been merged (both BaseMaterials and RenderTargets): RGBA RenderTargets are being used, velocity mapped on RG channels, density on B channel

RGBA RenderTargets: RT_Composite, RT_Advec  on, RT_Painter

B.  Dedicated Divergence RenderTarget is eliminated. The divergence Material is wri  ng its

output to RT_PressureDivergence, GREEN channel.

C.  Classic Ninja Pressure solver replaced with a new version (v2) --- v1 solver is s  ll

available: NinjaLiveComponent /LivePerformance /UsePressureSolver1 FLAG

Solver v2 does not perform repeated itera  on: calculates the pressure field in two steps - so pressure calcula  on is not a "subcycle" anymore - but two regular steps in the

Live 1.2: 140 FPS

vs

Live 1.3: 230 FPS

execu  on chain. The divergence and pressure materials are using bi-channel RG-RenderTargets, with pressure mapped on R, divergence on G.

Level 13: SmokeChamber test scene   TWENTY 720px fluidsim containers

Live 1.2: 100 FPS	vs	Live 1.3: 190 FPS	RG RenderTargets: RT_PressureDivergence, RT_PressureDivergenceTemp

22.3   Original fluidsim pipeline descrip  on (valid for Live 1.0 - 1.2):

NinjaLive rendering pipeline is a chain of Blueprint controlled Materials  >>>  wri  ng to RenderTargets  >>>  set up to form two cross-talking feedback loops: density & velocity. The numbers below are references to the included Chart (No.2).

1,2,3,4

Sim could be driven by textures, materials, user gestures, camera input and most importantly: objects overlapping / colliding with the simula  on area. Collision data (posi  on and velocity) is transformed from WorldSpace to SimSpace and fed to the CollisionPainter Material, wri  en to a RenderTarget then injected to the simula  on feedback loop.

5,6

Compositor Materials could mix / add / subtract various inputs ( eg. genera  ng a cloud via Material Input (No.3) and subtrac  ng Collision Input (No.2) as if colliders are "cu   ng through" the cloud). Note: Material 6 is merged with Material 11 for prac  cal reasons.

7,8

Following the merging of inputs by compositor materials, the data is ready for advec  on: transport in velocity defined direc  on. Technically, we do this by modifying the standard texture sampling UV coordinate grid (two linear gradients) with the velocity map - to offset sampling posi  on. Subtrac  ng the scaled velocity from UV causes a given grid-cell to read it's content from inverse flow defined direc  on and distance - the sampled density and velocity informa  on tends to "move over the grid" as the simula  on advances.

Note1: why do we sample from the inverse direc  on? Have a look at Reference No.2 - key insights on the sca  er vs gather method, related to this ques  on Note2: the velocity field drives its own advec  on (besides density advec  on).

9 - 10 - 11

This operator group plays an important part in construc  ng and maintaining simula  on velocity fields - by extrac  ng divergence, using it to generate pressure field gradient and feeding it back to the simula  on cycle.

9

Divergence is a key concept, that could be loosely defined as: "the rate at which 'density' exits a given region of space" (Ref.1) or "a measure of how much fluid enters or leaves each

(simula  on) cell" (Ref.3), or the amount of local "compression and expansion" (Ref.2). Precisely: "the volume density of the outward flux of a vector field from an infinitesimal volume around a given point (Ref.4).

It is important to note that our fluid model describes the dynamics of incompressible fluids - and we expect constant pressure / homogeneous density over the field. Confusingly, divergence and the data generated from divergence is used in a context that makes it interchangeable with the concept of pressure - but it describes a local property of the velocity field (we could call it "local tension").

Technically, we calculate divergence by sampling the fluid velocity vectors in the four adjacent cells around a given texel, performing subtrac  on of horizontal & ver  cal neighbours separately (right-le  , top-bo  om) and adding the results. The divergence of a vector field is a scalar field with no direc  onal informa  on, only float data (like a height map).

Examples for possible null states: (1) neighbour vectors are null. (2) neighbour vectors point to the  same direc  on / same length, the result of both H/V subtrac  on is zero. This could be the case when a cell is in a homogeneous unidirec  onal flow. (3) the sum of horizontal and

ver  cal components cancel each other, eg. horizontal vectors poin  ng inwards, ver  cal vectors poin  ng outwards. In all cases (1-3) influx == ou  lux, no divergence. Nega  ve /

Posi  ve divergence values indicate non-equilibrium state, a tension where influx =! ou  lux, somewhat analogous to local (texel neighborhood) pressure building up or decreasing.

We have at least two reasons to reduce divergence of the velocity field. Theore  cally: we need to keep pressure (near) constant. Prac  cally: velocity field modulates our sampling offset in

the advec  on module - and a highly divergent field with non-gradually / irregularly changing neighbour values causes visible "texture stretching" ar  facts - or noise at the extreme.

10

Pressure cycle

One way of reducing local divergence is spa  al dispersal / relaxa  on. We are employing the Jacobi method to perform this, a recursive func  on: (1) adding four adjacent neighbour texel values, (2) adding nega  ve divergence, (3) dividing the total sum by the number of neighbours (in this case four) and finally using the func  on output + raw divergence as input for the next cycle. Note: subtrac  ng divergence and adding nega  ve divergence results the same: compressed areas are represented by posi  ve values and expanded areas with nega  ve values during the itera  on.

In common sense, the itera  on process could be compared to "diffusion": peak divergence values (both nega  ve and posi  ve) "spread out", forming "ramps" or "gradients". A  er N

itera  on, we have a scalar field with "dispersed divergence"... s  ll hesitant to say it is pressure (we could call it tension). Note: the dispersal turns (integrates?) local tension to a global field.

11

EXPERIMENTS

To validate the above statements, I have performed experiments using Ninja core - and found very similar experimental setups at the SideFX homepage (Ref.2) - published as part of a Pyro tutorial, Pressure projec  on part - please have a look.

1.  Forcefully injec  ng outward poin  ng velocity field:

posi  ve divergence builds up mid field   (like "pressure drop")

2.  Forcefully injec  ng inward poin  ng velocity field:

nega  ve divergence builds up mid field	(like "pressure increase")

3.  Forcefully injec  ng a posi  ve divergence spot mid-field:

inward poin  ng velocity field builds up

4.  Forcefully injec  ng a nega  ve divergence spot mid-field:

outward poin  ng velocity field builds up

Gradient	(note: this material is merged with No 6. for prac  cal reasons)	Observa  ons:

Using the scalar field as an input, we calculate the gradient (the direc  on of ascension on the "pressure landscape") by performing subtrac  on on the horizontal / ver  cal neighbours separately - and using these par  al results as components of a direc  onal vector. An example for horizontal direc  on: if the pressure is increasing from le   to right (right neighbour is larger than le  ), value is posi  ve, and if the pressure is increasing from right to le   (le   neighbour is larger than right), value is nega  ve.

Using inverse gradient gives us the direc  on of "descent" on the pressure landscape (instead of ascension) - and this seems matching the intui  on: the direc  on of advec  on is the inverse of pressure gradient, flow goes from high pressure zones towards low pressure zones. By adding the inverse gradient to the exis  ng velocity field each simula  on step we are (1) facilita  ng a divergence driven advec  on and (2) correc  ng (countering, elimina  ng, dampening) field divergence at the same   me.

●  Divergence and velocity are connected in a feedback loop and have

induc  ve effect on eachother

●  If we remove the forceful maintenance of a certain extreme, the effects seem to

cancel out each other and the system returns to equilibrium state

But who comes first in a "realis  c" situa  on? By realis  c we mean adding moving colliders to an otherwise neutral field (eg. throwing a pebble in a pond). Modifying the field directly is not a realis  c situa  on. We could decide this ques  on using an experimental setup

(also demonstrated at sideFX page, Ref.2)

1.  Imagine a 2D null velocity field, for direc  ons we use up/down, le  /right nota  on

2.  All field vectors zero and an object mid field, s  ll / fixed posi  on

3.  The object suddenly starts to move upwards, with constant speed

4.  The movement injects upward poin  ng (posi  ve, ver  cal) velocity to the field.

5.  At the ini  al simula  on step (when the object first moved) the texels immediately above the moving area show non-zero divergence: comparing their neighbour vectors, we have influx from below, and no ou  lux at the top. Divergence is nega  ve - we are compressing.

6.  The area below the object shows posi  ve divergence (decompression / expansion): the bo  om row that has been "le   behind" by the object is filled with upward poin  ng velocity - and the row below that is zero, and this config gives back posi  ve divergence values.

7.  Intui  vely, we would expect these states to propagate

8.  Indeed: the Jacobi dispersal immediately (in the first round of larger simula  on cycle) makes a gradient from the sharp (single row) high divergence values both in the upper and bo  om areas. As we invert this gradient to see descent direc  on, we find that both regions (above and below the moving object) contain upward poin  ng direc  onal vector components, but the direc  on of horizontal components is inverted: the upper zone propels advec  on "up and out" while the bo  om zone propels advec  on "up and inward" - just like in real fluids.

As SideFX folks put it: "Pressure projec  on adds veloci  es around its sides to channel the flow back to the bo  om of the circular region." - see image link

CONCLUSION

In prac  ce, it is the divergence-pressure-gradient operator group that constructs extended, proper velocity field during the process of pressure projec  on.

I would like to say thank you to Morten Vassvik :)

REFERENCES

1. GPU Gems, Chapter 38

h  ps://developer.nvidia.com/sites/all/modules/custom/gpugems/books/GPUGems/gpugems_ch38.html

2. SideFX, Understanding how pyro works,

h  ps://www.sidefx.com/docs/houdini/pyro/background.html

3. Content-Driven Mul  pass Rendering

h  ps://www.youtube.com/watch?v=QGIKrD7uHu8

4. Wikipedia, Divergence

h  ps://en.wikipedia.org/wiki/Divergence

5. Divergence and Curl

h  ps://www.youtube.com/watch?v=rB83DpBJQsE

6. Wikipedia, Flux

h  ps://en.wikipedia.org/wiki/Flux

23. Using Sequencer to control NinjaLive Actors

Related example content: Level 5, Stage 4 (added to Live 1.0.0.4) Related Tutorial Video: LINK

NinjaLive interac  ons and behaviour could be keyframed by UE Sequencer.

In general, we could animate (1) effectors - any agent that triggers NinjaLive response and (2) sim parameters - internal variables that change fluidsim behaviour and

(3) output material instance parameters. Case “3” is not described in this document.

23.1 Effectors

An object / mesh could be effector if...

A. "Generate Overlap Events" op  on at Actor Details /Collision is flagged ON

(by default, it is OFF) --- See 10.5

B.  their "Collision Object Type" is set to one of the supported types

( WorldSta  c, WorldDynamic, Pawn, Vehicle, Destruc  ble, PhysicsBody )

C.  the class filters of the triggered (overlapped) container (NinjaLive Actor) are set to

respond to the type of the given object. See 10.1.6

If an effector meets the above criteria and is keyframed to collide / overlap with a NinjaLive container it should trigger fluid response.

(Make sure you have properly set "Interac  on Volume" size) --- see 10.1.5, 21.2.2

Note 1: Live generates fluidsim at run  me. To preview effector-fluid interac  on, you should run the game (press play in editor). In case you are running Live in a Viewport, you could fit Sequencer to a nearby panel and edit keyframes / move the   me-slider while the sim is running.

Note 2: One method to run the already keyframed sequence automa  cally at Play (and set the sequence to repeat) is to edit the Level blueprint. See Screenshot: LINK

23.2 Parameters

NinjaLiveComponent is using many params - most of them are set only once, at ini  alisa  on - these can not be animated/keyframed (typically system variables). A subset of params (typically fluidsim params) are being refreshed every   ck - and could be efficiently animated. These params also appear on the Preset Manager UI. See 21.1 for a complete list.

Important:   NinjaLiveComponent blueprint params are by default

NOT accessible for the sequencer.

A.  To make a given fluidsim param accessible for the sequencer:

open /Content/FluidNinjaLive/NinjaLiveComponent blueprint, look up the param in the "Variables" list, select it, go to the Details panel (in the blueprint editor) and

flag "Expose to Cinema  cs" = ON. Compile and Save the blueprint.

B.  To animate "Exposed" fluidsim params, add a given NinjaLive Actor to the sequencer

Track list (+), then add NinjaLiveComponent as subtrack (+), then add the (pre-emp  vely) exposed param as subtrack (+)

23.3 Cinema  cs Rendering via MOVIE RENDER QUEUE (UE 4.26+)

NinjaLive is working fine with the new UE 4.26 Movie Render Queue With the predecessor Render to Sequence it does not work!

A step-by-step guide

1.   Edit /Plugins /MovieRenderQueue: ENABLE PLUGIN

2.   Open the level that you would like to render

( NinjaLive_Level08_Demo_Roots is ideal for tes  ng: single container, no proximity ac  va  on )

3.   Select "BP_NinjaLive_U  li  es" on level and on the Details Panel:

switch OFF the "Possess Nearest Pawn" flag

4.   Content Browser: right click /CreateAdvancedAsset /Anima  on /LevelSequence

5.   Place Actor Panel: drag a camera on level ---> set the camera to the needed posi  on

6.   Content Browser: double-click on the level sequence

7.   In Sequencer: right click, "Actor to Sequencer"

---> add the recently placed Camera from the Level

8.   Viewport, Top-Le   corner UI /Perspec  ve roll down menu: select Camera Actor

9.   Window /Cinema  cs /MovieRenderQueue (MRQ)

10. In MRQ, click on the green "Render" roll down menu

--> add the previously created Level Sequence

11. Adjust Output Folder and Config - if needed

(by default: JPEG sequence, to "/Saved/MovieRenders/..")

12. Check 23.4: QUALITY NOTES

13. Press green "Render (Local)" in the Bo  om-Right corner

14. A  er a few seconds of ini  aliza  on, MRQ renders the image sequence to the provided

folder

23.4  IMPORTANT NOTES ON QUALITY

Note 1: on many levels, sim containers are Proximity Ac  vated (work only when the spectator/pawn is close) - and the cinema  c camera does not necessarily trigger the proximity sensor. In this case, your sim container is passive. Disable proximity sensor at:

NinjaLive details panel /LiveAc  va  on

Note 2: many   mes, distance based LOD is enabled on sim containers (lowers quality) To make sure you render the sequence using the best available quality, disable LOD at: NinjaLive details panel /NinjaLiveComponent /LivePerformance /LOD bool flags

Note 3: ninja is performing calcula  ons in the 60 FPS range. When the Movie Render Queue is set to 30 FPS, it hurts ninja visual quality. Set MRQ rendering FPS to 60 and (if needed) achieve lower frame rates by skipping odd/even frames when composi  ng the output.

Related chapter in the Manual: Chapter 23, Sequencer (LINK)

Related tutorial videos:

FluidNinja LIVE - Using SEQUENCER to animate Objects and Simula  on Containers (LINK) Improve Your Renders With Unreal Movie Render Queue (LINK)

24. Volumetrics

Example content: Level 23-28 (added to Live 1.2) + Level 30-32 (added to Live 1.4) Tutorial Videos: LINK1, LINK2  + Live 1.4 Volume Smoke quick preview: LINK3

NinjaLive could drive 3D volumetric systems with real   me 2D fluid sim data: users could set up responsive fog/smoke containers on level and paint dynamic cloud structures. Simula  on density is used as a height-field, combined with 3D noise that is advected by sim velocity. Output is a true 3D volume, shaded by scene ligh  ng. Comparing the three available systems:

24.1   INTEGRATION

Driving Volumetric Systems is a new, experimental feature - for this reason setup components are purposely kept external (not integrated to Ninja blueprints) - so users could customize setups without blueprint modifica  ons. Setup components:

1.  On-disk (pre-generated) RenderTargets - to push fluid data towards Volumetric Systems

Added to NinjaLive version Lowest supported UE version UE na  ve (built in) system Material Domain

Volume bounds defini  on Spa  al usage

Self shadows

FOG

1.2 4.23 True Volu Mesh Local

-

SMOKE

1.4

4.23

-

Volu Mesh Local True

CLOUD

1.2 4.26 True Volu UVW Global True

2.  Manually defined volume bounds (Fog: Sta  cMesh boxes, Smoke: Blueprints )

3.  Volumetric domain BaseMaterials and Material Instances

4.  Special UE Actors placed on level, needed to u  lize UE Volumetrics (Exponen  alHeightFog Actor, VolumetricCloud Actor)

Wri  ng to external RenderTargets is managed on the following Ninja UI panels: NinjaLive Actor Details Panel /NinjaLiveComponent /LiveGeneric /...

Lit by direc  onal light Lit by point light Receives Shadows Frustum aligned grid*

- True True True

True

True

-

-

True

-

-

-

DrawInternalRenderTargetToExternal InternalRenderTargetsToExport ExternalRenderTargets

(enables the feature) (array of sources) (array of targets)

Volumetric system EXAMPLES are using the following premade RenderTargets:

Note1: UE na  ve volume fog is implemented as *froxels - voxel array stretched by the cone-shaped camera frustum.  This means the rela  ve (per square unit) resolu  on of the grid is ge   ng lower by distance. For this reason, NinjaLive VolumeFog Base Material contains a “fade by distance” func  on - highly advised to use it. Note 2: the spa  al and temporal resolu  on of fog could be adjusted by console commands, listed on level 23 r.volumetricfog.gridpixelsize is the most important (default = 8) - set to 5 by the Level Blueprint on LV23

FOG: CLOUDS: SMOKE:

RT_VolumeFog1-4 RT_VolumeClouds1-8 RT_VolumeSmoke1-2

24.2   DEFINING VOLUME BOUNDS & POSITION

A. Volumetric Fog: a simple Sta  cMesh Box is placed on level, equipped with Volumetric Domain material, reading NinjaLive output RenderTargets - see "VoluCube" meshes listed in the WorldOutliner and placed on Level

B.  Volumetric Clouds: bounds and posi  on are defined within the material (no mesh

needed), using UVW coordinate offset and scaling

- Select "VolumetricCloud" Actor on Level

- Go to Actor Details Panel, and double-click on "Cloud Material"

- In the Material, locate CloudU-Offset, *V-offset and *Al  tude params

C.  Volume Smoke: bounds are defined by a level-placed blueprint

Blueprint name: VolumeSmokeContainer

A debug box visualizes volume extension. To resize the volume, simply scale the level placed blueprint actor (use uniform scale).

Volume loca  on is anchored to the blueprint actor

24.3   TILING SIM SPACE: FULL SKY / GROUND COVERAGE

Live 1.2.26.5 contains a new feature: sim space   ling (by default OFF) Usage and setup demonstrated on a new level: 24C

The feature enables us to compute only a small part of the sky (or ground-fog) - and use this part as a “  le” - the sim space is wrapped - so, we could cover the whole area (sky or ground) using repeated pa  erns.

Tiling could be set up / switched ON by tweaking the following params:

1.  NinjaLiveComponent /LiveGeneric /SimAreaClamp bool flag - set to FALSE

2.  PresetManager /SimAreaBorder se   ngs - set all params to ZERO

3.  VolumeFog and VolumeCloud material instances /Tiling bool flag - set to TRUE

More info: on se   ng up sim-space   ling: Level21 / Stage3 Tutorial videos on   ling - specific: LINK1 / generic: LINK2

24.4   STEP-BY-STEP GUIDE: LINKING VOLUMETRICS AND NINJA

NinjaLive is performing real-  me fluid simula  on. The sim buffers could be used to drive volumetrics. For example: sim density could be used to define volume density, height and ex  nc  on. Sim velocity field is ideal for driving the flow of detail-noise.

To do: we are wri  ng ninja internal simula  on buffer(s) to external RenderTargets and set a given volumetric material to read these RenderTarget(s) as input

1.  Create one or more RGBA RenderTargets in advance

These are going to serve as a data-bridge to Volumetrics. Right click in the Content Browser, go to "Materials and Textures", choose "RenderTarget".

2.  Set ninja to write the RenderTarget(s)

At: NinjaLive Actor / NinjaLiveComponent /LiveGeneric

a.  switch on "Draw Internal RenderTargets to External"

b.  chose which buffers to export

c.  pick an exis  ng (previously created) empty RenderTarget to write

3.  Place special UE Actors on level

a.  For VolumeFog: place a single Exponen  al Height Fog Actor on Level.

Set the "Volumetric Fog" op  on to ENABLED.

Important: VolumeFog materials ARE NOT DEFINED in the Fog Actor

4.  Once special actors placed on level, RenderTargets created and ninja is set up to

write them... tell the volumetric materials to read the RenderTargets

a.  In the case of VolumeFog, the volume-materials are applied on the level-placed Sta  cMeshes (see Level 23, browse "VoluCubes" in the WorldOutliner). Important: mul  ple meshes could be placed on level, and each mesh could be equipped with unique material. You could place your own mesh on level (any Sta  cMesh will do). Cuboids are prac  cal + could be scaled non-propor  onally. It is suggested to duplicate one of the example material instances, and provide the duplicate with new RenderTarget Input - then, apply the material on the cuboid mesh.

Do not forget to switch on "VolumeFog visualisa  on" in the editor (Ctrl+F), in UE 4.26.2 it is switched off by default.

b.  In the case of VolumeClouds, the volume-material is directly provided in the VolumetricCloud Actor (no need to place a volume-mesh). You could set scale, offset and many other params in the material. Do not forget to set the input RenderTargets!

c.  In the case of VolumeSmoke, the used Volume Material Instance and the input RenderTarget are defined on the NinjaLive_VolumeSmoke blueprint details panel. Depending on the param flags in the VolumeSmoke Material Instance, we should dis  nguish SIX material states (marked with No 1-6).

b.  For VolumeClouds: place a single VolumetricCloud Actor on Level. Important: VolumeCloud material (one per level) IS DEFINED in the VolumeCloud Actor

VolumeNoise:   FALSE VolumeNoise:   TRUE

UNLIT

1

4

POINT-LIT

2

5

DIRECTIONAL-LIT

3

6

c.  For VolumeSmoke: place NinjaLive_VolumeSmoke blueprint on Level.

Cases 1-4 are fast, ready for real-  me gaming usage. Cases 5-6 are somewhat slower, mainly developed for Cinema  c / NextGen usage.

24.5   ON DEMO LEVEL CONTENTS

Level   23

Volume Fog

Levels 24 ABC, 25, 26, 27, 28	Volume Clouds

Levels 30, 31	Volume Smoke

Spa  al arrangement

On Levels 23, 26, 30 and 31 the sim containers (detec  ng collision) occupy the same loca  on where the driven volume is. This setup enables actors to seemingly interact with the volumes. In fact, they interact with sim containers - and these drive volumes.

On levels 24A, 24B, 24C and 25 sim containers and volumes are spa  ally detached: (containers are running "somewhere else" / "behind the stages'') - this setup style is used when we do not need direct interac  on between volumes and scene actors / we would like to "playback" a predetermined sequence of events - eg. a cloud vortex forming when summoning the final boss.

For “background” containers, we also need “non-camera based” line tracing - a new feature, introduced in Live 1.2 ---> explained in Chapter 10.2.6 + see Level 2C

Sim output display

In a classic NinjaLive setup, TraceMesh has two separate func  ons:

(1) It is capturing collision data	(intersec  ng with the line tracer)

Interac  on

In a classic NinjaLive setup, TraceMesh captures collision input - and displays sim output at the same spa  al loca  on - we always see interac  ng objects and the simula  on together. This setup allows us to conveniently use the CAMERA as line tracing source. Volumes could be easily detached from the sim container and display sim output "somewhere else": in these cases, the camera can not be used as line tracing source. Reason: when we see the volume, we don't necessarily see the sim container - eg. watching a skydome with clouds, while the container is "behind the scene".

To resolve the problem, NinjaLive 1.2 introduces User Defined Line Trace Source.

(1) UI:NinjaLiveComponent Details Panel /LiveInterac  on /UseCustomTraceSource

(2) Dedicated chapter in this manual: 10.2.6 / Usage examples: on Level 2C

(3) Level 24-26 help texts explaining usage in situ (on the level)

Custom LineTrace Source combined with disabled LOD results in containers that could work “behind the stages” / away from the volumes - while not visible.

No interac  on

(2) Simula  on is mapped on it

(it is visualizing simula  on output)

There are cases when we don't need interac  on at all: simula  on input is a texture (sta  c) or a material (animated), user interac  on and overlap detec  on is completely

In volumetric setups, simula  on output is rendered using the volume - so TraceMesh loses its role as a display (like on Levels 24-25-26)... or, it could be combined with the volume (both TraceMesh and Volume are displaying sim output) - like on Level 23.

switched off (at NinjaLive Actor Details panel /LiveInterac  on), simula  on is running without any interac  on with the scene ----> See Level 24B, 24C

25. Detail Maps

A feature to add dynamic, flowing details to low and medium resolu  on simula  ons at minimal cost. Introduced to NinjaLive 1.3

We are using sim velocity buffer as a flow map to advect a user defined map (typically procedural noise) - and mix the dynamically flowing details to the na  ve simula  on output

Included examples:

- Level 29	adding cell noise to flame or combus  on type fluids

- Level 11	adding a cloud noise to a smoke-sim

- Level 10B, Stage1	adding grainy "sand like" noise to viscous fluids

Accessible at:	Output Material Instance /FlowMap parameter group

Concept behind Detail Maps:

Ninja fluid-sim pipeline is a collec  on of various data types like density, velocity, pressure, divergence. Density is generally handled like the "final product" while others considered as "by-products". How about u  lizing sim "by-products" for something useful? Pressure is ideal for op  cal and geometric distor  on (see Level21 Stage6), velocity could be used to drive

par  cles (see Level 20A,B) or: to drive texture advec  on - tradi  onally called "flow mapping". The flowmap concept: we are using two iden  cal copies of a sta  c texture and distort the image-pair using a shi  ed period oscilla  on. Distor  on is driven by a velocity map. A velocity map is a field with direc  onal vectors, ideal to tell par  cles / texels "which direc  on to go".

Ninja already u  lizes flowmaps - this is how we make volumetric noise flow on clouds - see Levels 24-28. Now, an other flowmap feature has been implemented: detail mapping

26. Controlling Live in real   me

To grasp the idea of real   me control, load any ninja tutorial level, place a Preset Manager, start the Game (Play) and use Preset Manager to (A) interac  vely set Fluidsim Parameters, (B) change Input Textures and (C) Output Materials.

See videos: changing Sim Params: LINK / changing Output Materials: LINK1,  LINK2

Similar control could be achieved by Game Logic (Blueprints, Code) or Sequencer.

●  Prac  cal example1: using NinjaLive Actor for an area FX. We would like to control the FX

params using sequencer - to create a choreographed Cinema  cs Sequence

See this video: LINK

●  Prac  cal example2: character ability FX is delivered by an embedded NinjaLiveComponent - we switch the fluidsim preset and the output material as the character uses various ability-FX

Control subjects in the NinjaLiveComponent blueprint could be grouped as:

1.  Could be modified instantly

2.  Could be modified by re-ini  alizing NinjaLiveComponent blueprint

1. Simple Variables (eg. floats, integers) could be modified instantly by accessing NinjaLiveComponent. Variables have "telling names" - and you could read about them using the Tool  ps and the Manual. Pls open up NinjaLiveComponent Blueprint: Preset Variables (for example) could be found under the "NONPUBLICLive Preset Variables" group. --- by code you can access them any  me. In the Blueprint that embeds NinjaLiveComponent (eg. a character) params could be directly accessed. For other (external) blueprints, the LIVE INTERFACE could be used. To modify a variable by Sequencer, you need to set "expose to cinema  cs" to TRUE. See Level 5, demonstra  ng how to use the Interface and Sequencer.

2. Asset-type-variables (like a DataTable with preset-values, or an OutputMaterial Instance) could be modified by re-ini  alizing NinjaLive. (Trivial example: the Preset Manager re-ini  lizes ninja when changing materials or loading a new preset.)

---> See image below: Preset File and OutputMaterial index changing - combined with RePlay

Swapping Materials: only those output materials could be accessed (and used for swapping), that are previously added to the OutputMaterials array at LiveComponent /LiveGeneric Swapping process: (1) changing the OutputMaterialSelected INDEX, then (2) re-ini  alize ninja

Please open NinjaLiveComponent Blueprint, and locate MODULE001. Check the RePlay node. As the label tells us: this event could be called remotely (eg. by an interface, or event dispatcher)

27. Mapping sim on 3D Mesh surfaces

See this video, demonstra  ng the usage of 3D meshes: LINK The technique is UE5 compa  ble.

NinjaLive is a 2D fluid sim. Besides the original (A) camera facing flat planes and (B) parallax mapping, new techniques are being added to enhance and spa  alize sim output.

Live 1.2 could u  lize (C) UE Volumetric Fog and (D) UE Volumetric Clouds (see Chapter 24). Live 1.4 introduces two more techniques:

1.  raymarching based translucent volumetrics (en  tled as "VolumeSmoke")

2.  polygonal mesh based opaque 3D surfaces

This video demonstrates the two techniques: LINK	+  you could read about VolumeSmoke

in Chapter 24. This chapter describes the mesh based solu  on.

Originally, Ninja used Parallax Occlusion Mapping (POM) to imitate depth. POM does not look good from a low (FPS) angle. Please have a look at LEVEL 10-B. The fluidsim containers on this Level are all using OPAQUE Output Material, which creates a discrete surface - the fluid surface is NOT at all fog like (it is non-translucent) - but opaque, shiny, reflec  ve - like blood, mud, snow.

The opaque surface property enables us to use real 3D meshes to display sim output:

by defining a pre-tessellated (hi-poly) grid as TraceMesh, and using simula  on density as a height-field to drive vertex World Posi  on Offset (WPO), we could dynamically distort the TraceMesh: the result looks good from low (FPS) camera angles as well.

27.1   Reconfiguring a sim container from POM usage to 3D sim mesh usage A STEP-BY-STEP GUIDE

1.  Select Container

2.  Select TraceMesh Actor Component

3.  Replace "NinjaLiveTraceMesh" (a single quad plane) with a tessellated plane

(eg. SM_plane_300x300)

4.  Enable "Cast Shadow" at the mesh component Details Panel

5.  Select NinjaLiveComponent

6.  Go to LiveGeneric param group, Locate the currently used OutputMaterial in the array

(checking the OutputMaterialSelected INDEX below the array helps)

7.  Locate "Parallax" param group, disable ParallaxMapping

(not needed when using a 3D sim mesh)

8.  Locate "MeshDistort" param group, enable MeshDistor  on, tweak params

(pay a  en  on to CLAMP!)

28. Moving in WorldSpace

Example content:  /Tutorial/Levels/	Level 04

/Usecases/Levels/ Usecase_001_WorldSpaceWater

28.2 Addi  onal se   ngs to tweak:

Ninja sim containers could be used two ways:

1.  Area FX: the sim is locked to a fixed WorldPos  on - like a puddle, a bonfire, a portal ...

2.  Character/Vehicle FX: the sim is a  ached to a moving Actor - like a torch with a smoke

trail, turbulence around a moving ship, dust caused by footsteps ...

When the sim container is travelling in WorldSpace, we need to modify the local sim according to global movement direc  on and speed.

28.1 Main methods:

A.  Offset ALL simula  on buffers inverse direc  on

Feature introduced in Live 1.5

Enables the already emi  ed fluid to "lag behind" the emi  er/source

B.  Modify sim VELOCITY buffer with the mo  on vector

This feature could push fluid towards or against moving direc  on

The key params are exposed at: Preset Manager UI /Fields sec  on /... ...  /All Fields Offset by Sim Area Mo  on

...  /Velocity Field Influenced by Sim Area Mo  on

The combina  on of A+B could result in interes  ng FX. For example, forcing the sim to lag behind, but also se   ng a nega  ve velocity that pushes it along moving direc  on results in a fluid that creeps / lingers behind the emi  er :)

A. Feedback (Preset Manager GUI /Sim sec  on)

Defines "Trail Life  me" - the   me needed to fade out the lingering fluid

B.  Drag (PresetManager UI /Brush sec  on)

S  rring/dragging effect of moving objects on the simula  on

C.  Puncture-modify

NinjaLiveComponent Details /LiveBrushSe   ngs /SimAreaMo  onEffectsBrushPuncture The Z component of sim area mo  on (perpendicular to TraceMesh) mul  plies PUNCTURE --- in prac  ce: when the sim container is moving towards/away from us, the "boiling" effect of puncture is more intense (by default) - and less intense when the param is set to ZERO

D. Latency

NinjaLiveComponent /LiveInterac  on /LegacySimAreaMo  onEffectsSimDensity

28.3 Warning:

FieldsOffset* is a key param --- needs to be tweaked carefully

*Preset Manager UI /Fields sec  on /All Fields Offset by Sim Area Mo  on

Different TraceMesh size / Different sim resolu  ons need different FieldsOffset values, eg.: the larger the tracemesh, the lower the FieldsOffset. Turbulence values are also sensi  ve: when working with HD sim, set Turb. < 1

29. Addi  onal Content: UseCase Packages

With NinjaLive version 1.5.26.1, the first round of adding core features has ended. Star  ng with 1 August 2021, we are focusing on producing prac  cal examples.

The examples are developed in small packages - and these packages are not automa  cally merged to / added to ninia main branch. Plan:

1.  we are developing the packages in union with the ninja-user community,

using the ninja discord as a communica  on and exchange pla  orm

2.  once a specific use-case is done, it is…

- showcased on a dedicated YouTube playlist - packed to a small zip file

- zip is distributed via ninja discord: on the #Announcements channel DISCORD INVITE LINK: h  ps://discord.com/invite/VpcyBQa77w

3.  the package is referencing core ninja assets

(eg. contains material instances - and base materials are in the core ninja project)

4.  packages are not cross referencing each other

(so you will never “miss one” in order to open an other)

5.  packages are numbered, and all located under /NinjaLive /Usecases

(to use a package, you only need to copy the zip content to /Content /NinjaLive)

When packages are becoming numerous, we are merging them in one step to the ninja main project -   me by   me (probably every month)

(so eventually all use case content becomes available via the marketplace project)

As an example, the first two UseCase packages has been included in the latest official NinjaLive version (1.5.23.2 / 1.5.26.2)

UseCase 001 is demonstra  ng how a single, camera (player) a  ached sim container with Sim.buffer WorldSpace Offset could be mapped on a large (infinite) water body to make the illusion that ninja is not a LOCAL, but a GLOBAL fluidsim. See preview here.

Package Assets: /FluidNinjaLive/UseCases/001_WorldSpaceOffset_FluidSurface/ Package demo level: /FluidNinjaLive/UseCases/Levels/Usecase_001_WorldSpaceWater.umap

UseCase 002 uses similar technology as 001, demonstra  ng heavy boat trails. The pack includes combo water materials that are both translucent and reflec  ve + an example for UE na  ve SingleLayerWater material. See preview here.

Package Assets: /FluidNinjaLive/UseCases/002_BoatTrail/ Package demo level:

/FluidNinjaLive/UseCases/Levels/Usecase_002_*.umap (4 maps)

UseCase 003 demonstrates how ninja could be used together with na  ve UE 4.26 water bodies (sea, lakes, rivers). Package available ONLY at Ninja Community Discord / Announcements channel: h  ps://discord.gg/VpcyBQa77w

UseCase 004 Tiling: running TWO ninja sim containers: (1) one to generate a   led pa  ern that could be used on infinite fields -  and a second (2) that handles local interac  on. Two sim outputs are merged by a Water BaseMaterial.

See preview: LINK

UseCase 005 Vehicle Trails: a WorldSpace ninja sim container generates area smoke/dust vfx for car tyres. Sim Container is a  ached to the car (while rota  on is locked) and hidden - output is used to feed a VolumetricSmoke Container with input data. Addi  onally, four small containers are a  ached to the wheels, to generate local smoke/flames. See preview: LINK

UseCase packages 4 & 5 have been included in a complete, new NinjaLive Release v1.5.26.3 - available via EPIC launcher and the Marketplace.

UseCase 006 Large Water Bodies: Rivers, Lakes, Whirlpools

This addon package demonstrates how mul  ple ninja sim outputs are composited together by a single material - to create large water bodies. We are using a   le generator (1) and a local solver (2) sim.

---> Youtube Demonstra  ons: SHORT, LONG

UseCase6 is released as a 10 Mbytes ZIP file - an ADDON for NinjaLive version 1.5.23.3 -- The ZIP file is available at the Ninja Community Discord #Announcements channel

UseCase 007 Small Water Bodies: Waterfalls, Pools, Fountains

This addon package demonstrates how local ninja simula  ons could be turned to small water bodies by u  lizing available sim buffers - pressure to distort geometry and calculate normals, velocity to drive detail maps (surface foam) and density to add large scale details. Buffers are integrated by a new BaseMaterial, capable to deliver reflec  on, refrac  on and translucency.

---> Youtube Demonstra  on: LINK

UseCase7 is released as a 15 Mbytes ZIP file - an ADDON for NinjaLive version 1.5.23.3 -- The ZIP file is available at the Ninja Community Discord #Announcements channel

-------------------------------------------------------------------------------------------------------------

A new, 35 mins tut video is released, explaining all technical details behind the recent, WATER related use case packages 1-7: LINK